## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Mechanical Engg./P/A

Subject: Machine Drawing-I(4033)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A straight line PQ has it end point ‘P’ 10mm above H.P and 15 mm infront of VP. The line is 50mm long and its front and top views are inclined at 60 and 45 respectively. Draw the projections of the line and find its inclination with the H.P and the V.P. |  |
| b) | Fig.(1) shows incomplete plan and auxiliary plan of an object. Complete the plan. |  |
| c) | A triangular lamina of side 25mm is resting on a corner on H.P such that the surface makes angle of 45 to the H.P and the side opposite to the corner on which it rests makes 60 to the V.P. Draw its projections. |  |
| Q.No.2. | A monument is in the form of frustum of a square pyramid of base 1.2 metre side, top 0.5 metre side and height 1.0 metre. An electrical connection is to be made along the surface of this monument between one corner of the base and diagonally opposite corner on the top. Find the shortest length of the wire required and show the position of the wire in the top and front view. | (10) |
|  | ### OR |  |
| Q.No.2. | A lamp shade is formed by cutting a cone of base 144 mm diameter and 174 mm height by a horizontal plane at a distance of 72 mm from the apex and another plane inclined at 30 to the H.P., passing through one extremity of the base. Draw the development of the shade. | (10) |
| Q.No.3. | Answer any one of the following: | (12) |
| a) | A vertical cylinder of 60mm diameter is penetrated by a horizontal cylinder of 50mm diameter. The axes of the horizontal cylinder is parallel to V.P and 10 mm infront of the axes of the vertical cylinder. Draw the intersection curves. |  |

………2/-

-2-

| b) | A vertical cone, base 60mm diameter and axis 70mm long is penetrated by a cylinder of 40mm diameter such that the axis of the cylinder it parallel to the axis of the cone. Draw the projections showing the curves of intersection when the axis of the vertical cylinder is 7mm from the axis of the cone and the plane containing the two axis is perpendicular to the H.P and inclined at 45 to the V.P. |  |
| --- | --- | --- |
| Q.No.4. | Draw according to the system of projection used for fig.(2) i) Sectional Elevation along A-A ii)Sectional side view along B-B iii) Top Plan. | (16) |
| Q.No.5. | Answer any one of the following: | (08) |
| a) | With the help of a suitable freehand sketch show the following types of sections: - Half Section - Offset Section - Revolved section - Aligned Section. |  |
| b) | Give conventional representation for the following: - Leaf spring with eyes - Worm and worm wheel - Concrete - Ball bearing. |  |
| Q.No.6. | Redraw the views given in fig(3) by proportionate free hand sketch and show appropriate welding symbols. Plate marked as 1,2,3 and 4 are to be welds as mentioned below. - Piece (1) to be welded to piece (2) by single J butt. - Piece (2) is welded to piece (3) by all round fillet arc welding. - Piece (1) is welded to piece (4) by square butt weld finished flush on top. | (17) |

…..3/-