## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Introduction to Thermodynamics(4334)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Psychrometric charts, Steam Tables and Mollier’s chart is

permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State at least three properties of liquids and solids that vary with alteration of temperature. Explain how this property can be used for the measurement of temperature. |  |
| b) | The temperature of water while entering an economizer is 20c, while leaving economizer is 85c. If the rate of flow of water is 300Kg per minute, how much quantity of heat is supplied to water in the economizer? |  |
| c) | Define i) Specific heat at constant pressure ii) Specific heat at constant volume. What is the ratio of these specific heats called as & what is its value for air? |  |
| d) | Define i) Latent heat ii) Superheat. |  |
| e) | Write any four classification methods of I.C. engines. |  |
| f) | Define the term ‘COP’ related to refrigeration. |  |
| g) | Distinguish between parallel flow & counter flow heat exchangers. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the second law of Thermodynamics as per i) Kelvin-Planck ii) Clausius. |  |
| b) | Derive characteristic gas equation of perfect gas. In what units is the constant R measured? |  |
| c) | 1m3 of air from a pressure 100KN/m2 absolute and a temperature 27c is compressed according to law PVn = constant till its volume is reduced to one-third of its original volume and the pressure is 400KN/m2 absolute. Calculate i) Index n ii) The rise in temperature of air iii) The work done on air iv) Heat transfer during the process. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In an ideal otto cycle the air at the beginning of Isentropic compression is 1 bar and 15c. The ratio of compression is 8. If the heat added is 1005 KJ/Kg during the constant volume process, determine - The maximum temperature in the cycle - The air standard efficiency - The work done per Kg of air - The heat rejected per Kg of air. |  |
| b) | An engine working on Diesel cycle has a compression ratio 15. The pressure and temperature at the beginning of compression is 1 bar and 15c respectively. The maximum temperature of the cycle is 2427c. Determine - Heat supplied & work done per Kg of air - Cycle efficiency - Maximum pressure - Cut-off ratio. | …2/- |
|  | -2- |  |
| c) | A two stroke cycle, 20cm bore x 28cm stroke, single cylinder petrol engine gives the following results on test: - Speed = 350 r.p.m. - Net brake load = 620 N - Diameter of brake drum = 1m - Oil consumption = 4.25 Kg/hr - Indicated Mean effective pressure = 275 KN/m2 - Calorific value of petrol = 43000 KJ/Kg - Air fuel ratio by weight = 32 - Temperature of air in Test room = 20c - Temperature of exhaust gases = 370c Calculate i) Indicated Power ii) Brake power iii) Indicated thermal efficiency iv) Brake thermal efficiency. Assume Cp = 1 KJ/Kg K for the exhaust gases. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the internal energy of 1 Kg of steam at a pressure of 10 bar when the condition of steam is - Wet havind dryness fraction 0.85 - Dry and saturated - Superheated, the degree of superheat being 50c The specific heat of superheated steam at constant pressure is 2.26 KJ/Kg K. |  |
| b) | Determine the condition of steam in each of the following cases: - At a pressure of 10 bar and temperature 250c - At a pressure of 8 bar and specific volume 0.22m3/Kg - At a pressure of 8 bar, total heat is 2805 KJ/Kg. |  |
| c) | Compare the relative merits and demerits of 4 stroke & 2 stroke cycle I.C. engine. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe with neat sketch working of a four-stroke cycle petrol engine. |  |
| b) | Sketch the Rankine cycle on P-V diagram and describe its application to steam turbine plant. |  |
| c) | What should be the properties of an ideal refrigerant? Describe the main refrigerants commonly used in practice. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A vapour compression refrigeration plant works between pressure limits of 5.3 bar and 2.1 bar. The vapour is superheated at the end of compression, its temperature being 37c. The vapour is superheated by 5c before entering the compressor. If the specific heat of superheated vapour is 0.63 KJ/Kg K, find the coefficient of performance of the plant. Use the data given below: Pressure bar; Saturated tempc; Liquid heat KJ/Kg; Latent heat KJ/Kg. 5.3; 15.5; 56.15; 144.9. 2.1; -14.0; 25.12; 158.7. |  |
| b) | With the help of a neat sketch, show that a refrigeration cycle is a reversed heat engine cycle. |  |
| c) | What is the heat loss through a wooden slab which has an area of 4m2 and thickness 7mm. The inner and outer surface temperatures of the slab are 20c and 7c respectively. Thermal conductivity of the wood is 0.112 w/mk? Also state Fourier’s law of heat conduction. |  |

***********