## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Mechanical Engg./P

Subject: Tool Engineering-II(Press Tools)[5077]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Design and draw a die to produce sheet metal components as shown in fig.1. The component is made of mild steel having ultimate shear strength of 700 N/mm2. Your design should include: | (50) |
| --- | --- | --- |
|  | 1.Press tonnage calculations |  |
|  | 2.Selection of economic strip layout |  |
|  | 3.Bill of materials |  |
|  | 4.At least two views of the tie. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 5 = 10 |
| a) | What is spring back? How is it controlled? |  |
| b) | Explain with a neat sketch “Bulging Dies”. |  |
| c) | What is “Bend allowance”? Explain. |  |
| Q.No.3. | Write short notes on any three of the following: | 3 x 5 = 15 |
| a) | Lancing operation |  |
| b) | Defects in deep drawn parts |  |
| c) | Wiping Dies |  |
| d) | Rubber Dies |  |
| e) | Stripper used in dies |  |

……2/-