## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Fabrication Technology & Erection Engineering

Subject: Welding Technology-I(4237)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give the classification of welding processes. |  |
| b) | Explain with neat sketches any two terms from the following: (i) V- edge preparation (ii) Weld slope (iii) Throat thickness. |  |
| c) | No flux is necessary while welding steel with oxy-acetylene flame. Give reasons. |  |
| d) | Explain in short mechanism of Arc blow. |  |
| e) | Differentiate between Stelliting and Hardfacing. |  |
| f) | Corrosive fluxes are used while welding Aluminium and their alloys. Give reasons. |  |
| g) | Give three important safety recommendations for Gas cylinders. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the working of a two stage Gas regulator. |  |
| b) | Explain the different types of metal transfers in arc welding. |  |
| c) | Explain the principle of working and procedure of soldering operation. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the oxy-acetylene flame welding set. |  |
| b) | Differentiate between Constant current and Constant voltage power source characteristics. |  |
| c) | Explain the procedure of vacuum brazing. Differentiate between Vacuum brazing and Furnace brazing. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Give the safety recommendations for a Gas cylinder Manifolding system. |  |
| b) | Explain the following terms in welding: |  |
|  | (i)Pre- heating (ii) Inter-pass heating (iii) Post- heating (iv) Post weld heat treatments. |  |
| c) | Write a note on “Silver Brazing.” |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the important of base metal cleaning in soldering. Explain the different methods used for cleaning. |  |
| b) | Give the formula to determine “Carbon Equivalent” of alloy steel. Explain its effect on their weldability. |  |
| c) | Explain with a neat sketch “Arc metal spraying process”. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | List 3 important objectives of surfacing. Write a note on “Hardfacing of earth moving equipments”. |  |
| b) | Explain the different difficulties encountered while welding cast Iron. |  |
| c) | Explain with neat sketches any two of the following weld defects. Also give their causes and remedies. |  |
|  | (i) Overlapping (ii) Porosity (iii) Defective weld Bead Geometry. |  |

***********