## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Mechanical Engineering

Subject: Power Engineering-II(4041)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the functions of the convergent portion, the throat and the divergent portion of a convergent-divergent nozzle with reference to flow of steam? |  |
| b) | What do you mean by ‘diagram efficiency’ in steam turbine? |  |
| c) | Why do we use a condenser in a steam power plant? |  |
| d) | Classify the I.C. engine based on any 5 ways of classification. |  |
| e) | How supercharging helps to improve power output of I.C. engine? |  |
| f) | Name any six applications of Refrigeration? |  |
| g) | On what criteria is gas turbine classified? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Dry saturated steam at a pressure of 10 bar is supplied to convergent and divergent nozzle and is delivered at a pressure of 0.20 bar. Determine the diameter at the throat of the nozzle? |  |
| b) | Following data was obtained from an impulse turbine: Blade speed = 200 m/sec Exit angle of moving blade = 25 Steam velocity = 500 m/ sec Nozzle angle = 20º Neglecting the frictional losses find, - Inlet angle of moving blade - Exit velocity and direction - Work done per kg of steam. |  |
| c) | What is compounding of steam turbine? With neat sketch, show the variation of pressure and velocity in pressure compounded steam turbine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a sketch ‘regenerative cycle’ for steam turbine power plant. |  |
| b) | Draw a neat sketch of a surface condenser and explain how the condenser efficiency is interpreted. |  |
| c) | What are the effects of air leakage in condenser? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the working of 2- stroke petrol engine. |  |
| b) | Explain the differences between spark Ignition engine and compression Ignition engine. |  |

………2/-

-2-

| c) | An engine working on the Otto cycle has a compression ratio of 8.5:1. At the beginning of the compression stroke the conditions inside the cylinder are: Pressure 0.93 bar and temperature 93ºC. The maximum pressure in the cycle is 38 bar. Determine the pressure and temperature at each of the following points in the cycle: - End of the compression stroke - Beginning of the expansion stroke - End of the expansion stroke - Air standard efficiency of the cycle. Assume index of adiabatic process as 1.4. |  |
| --- | --- | --- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch and explain the valve timing diagram for four stroke diesel engine. |  |
| b) | The following particulars refer to a two stroke diesel engine. Cylinder diameter: 20 cm Stroke:30cm The diameter of brake wheel : 1m Indicated mean effective pressure : 2.8 bar Effective load on the brake : 64 kg Speed: 400 r.p.m. Diesel consumed : 4.6 kg/hr Calorific value of fuel: 44000 kJ/Kg. Calculate: i) Mechanical efficiency ii) Brake thermal efficiency. |  |
| c) | Draw the schematic diagram of a vapour absorption refrigeration system and explain the working of this system. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw T-S and P-H diagram for vapour compression refrigeration cycle when the condition of refrigerant entering the compressor is super heated. |  |
| b) | Describe with neat sketch the working of closed cycle gas turbine. |  |
| c) | What are the advantages and disadvantages of gas turbine over I.C. engine? |  |

***********