## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Mechanical Engg./P/A/SB/TD

Subject: Strength of Materials(4032)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the stress strain curves for ductile and brittle materials. |  |
| b) | Differentiate between gradual loading and sudden loading. |  |
| c) | Explain the terms: i)Neutral axis and ii)Bending moment. |  |
| d) | Explain the term ‘Buckling’ in columns. |  |
| e) | List down the assumptions used in theory of simple bending. |  |
| f) | Define the terms: - Moment of resistance and - Lever arm. |  |
| g) | Define a ‘Mohr’s circle’. Give a neat sketch of Mohr’s circle totally labelled. |  |
| h) | Define the terms: - Shear load - Shear strain - Modulus of rigidity. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | A member ABCD is subjected to point loads P1, P2, P3 and P4 as shown in fig. Calculate the force P2 necessary for equilibrium if P1 = 45 KN, P3 = 450 KN and P4 = 130 KN. Determine the total elongation of the member E = 2010 KN/ mm2. |  |
| b) | A steel cylinder is compressed in a copper tube. The cylinder and the tube are compressed between rigid parallel plates. Find the stresses in steel and copper and also compressive strain. P = 450 KN, d = 100mm, D = 200mm, D = diameter of a copper tube. ES = 210 KN/mm2, Ec = 110 KN/mm2. |  |
| c) | A bar of steel 32mm in diameter was subjected to a tensile load of 68 KN and the measured extension on a 20cm gauge length was 0.015cm and the change in diameter was 0.000380cm. Calculate Poisson’s ratio and values of three modulii. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | A sliding weight of 300N falls onto a collar provided at the lower end of a vertical steel rod fixed at the top. If the length of the rod is 2m and the area of cross section is 600 mm2, determine the height of fall, when the instantaneous stress is limited to 150MPa. E = 2 x 105 MPa. |  |

………2/-

-2-

| b) | At a point in a strained material, there are two planes at right angles to each other on which normal stress of 75 N/mm2 (tensile) and 45 N/mm2 (Compressive) are acting accompanied by shear stress. If the major principal stress is 105 N/mm2 (tensile), calculate the shear stress, minor principal stress and maximum shear stress and the planes on which it acts. |  |
| --- | --- | --- |
| c) | For an element under stresses P1= 100 N/mm2 (tensile), P2=0, shear stress = 20 N/mm2. Find graphically obliquity of the principal plane to the horizontal axis. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | A beam 2.5m long is simply supported at the two ends, and carries a point load of 40 KN at the centre, and a longitudinal axial tensile load of 100 KN. If the section of the beam is rectangular, 300mm wide and 500mm deep, find the maximum and minimum stresses at the mid span section of the beam. Neglect the self weight of the beam. |  |
| b) | A column 75mm x 75mm has a circular cavity 37.5mm in diameter at the centre. Find the maximum value of load that can be applied at an eccentricity of 10mm with respect to the xx axis in order that the maximum stress may reach 80N/mm2. |  |
| c) | Derive the relation for Core or Kernel of circular section. |  |
| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| a) | A thin cylindrical shell of thickness 5mm and diameter 350mm is subjected to an internal pressure which produces a strain of 1/2500 in the diameter. Find this internal pressure and the consequent hoop and longitudinal stresses. Take E= 2 x 105 N/mm2 and 1/m = 0.3. |  |
| b) | Determine the diameter of a solid shaft, which will transmit 90KN at 160 rpm if the shear stress in the shaft is limited to 60 N/mm2. Find also the length of the shaft, if the twist must not exceed 1 degree over the entire length. G = 8 x 104 N/mm2 |  |
| c) | A steel shaft transmits 105 KW at 160 rpm. If the shaft is 100mm in diameter, find the torque on the shaft and the maximum shear stress induced. Find also the twist of the shaft in a length of 6m. G = 8 x 104 N/mm2. |  |
| Q.No.6. | Answer the following: |  |
| a) | A simply supported beam span 9m is carrying a u.d.l. of 18 KN/m over a length of 4m on left hand portion of beam. Draw shear force and bending moment diagrams. Also calculate maximum bending moment and the point where it exists. | (8) |
| b) | A cast iron beam 40mm wide and 80mm deep is simply supported on a span of 1.2m. The beam is carries a point load of 15 KN at the centre. Find the deflection at the centre. E= 1.08 x 105 N/mm2. | (4) |
|  | ### OR |  |
| b) | A rolled steel I section is to be used as a column 4m long with one end fixed and other end hinged. Find the safe axial load on the column allowing a factor of safety of 3. Fc= 320N/mm2  = 1/7500 Properties of rolled section are: Ix = 5.1316 x 107 mm4 Iy = 3.345 x 106 mm4 | (4) |

***********