## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Civil Engg./CT

Subject: Soil Mechanics(4012)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 5 x 3 = 15 |
| --- | --- | --- |
| i) | Explain the significance of Soil Mechanics in Civil Engineering. |  |
| ii) | Define the terms: - Uniformity Co-efficient - Co-efficient of curvature. |  |
| ii) | Define the terms - Voids ratio - Dry density - Specific gravity. |  |
| iv) | Differentiate between compaction and consolidation. |  |
| v) | Explain unconformity of rocks. |  |
| vi) | Enlist the objects of soil stabilization. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| i) | Enlist and explain the different factors affecting permeability. |  |
| ii) | Define the term consistency of soil. Explain different states of consistency of soil. |  |
| iii) | A falling head permeability test was performed on silty sand. The time required for the head to fall in the stand pipe from 80 cm to 40 cm is 5000 seconds. The cross-sectional area of stand pipe was 125 sq.mm. If the height and diameter of the sample is 150mm and 80 mm respectively. Find the value of co-efficient of permeability (k). |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| i) | Explain different methods of compaction and their suitability. |  |
| ii) | With a neat sketch explain the laboratory consolidation test. |  |
| iii) | Determine the maximum dry density and optimum moisture content from the following Proctor Test results. |  |
|  | Sr. No.; 1; 2; 3; 4; 5. Water content %; 13.0; 20; 26; 36; 42. Density gm/cc; 1.60; 1.90; 1.98; 1.85; 1.60. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| i) | In a triaxial xield test following results are obtained. |  |
|  | Test-I; 1 = 120 KN/m2 , 3 = 60 KN/m2. Test-II; 1 = 180 KN/m2 , 3 = 100 KN/m2. |  |
|  | Calculate its cohesion (c) and angle of internal friction by plotting the above results. |  |
| ii) | Explain the procedure of plate load test. |  |
| iii) | With the aid of neat sketch explain the direct shear test. |  |

………2/-

-2-

| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| --- | --- | --- |
| i) | Explain the Rankin’s theory of earth pressure. |  |
| ii) | Explain any three types of folds with a neat sketch. |  |
| iii) | Enlist the methods of soil stabilization and explain any one method. |  |
| Q.No.6. | Write a short note on any four: | 4 x 3 = 12 |
| i) | Soil as a three phase system |  |
| ii) | Disturbed an undisturbed soil sample |  |
| iii) | Spring analogy |  |
| iv) | Bore hole log details |  |
| v) | Quick sand condition |  |
| vi) | Mechanical stabilisation |  |

***********