## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Civil Engg./CT/M

Subject: Hydraulics(4025)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Mass density ii) Specific gravity iii) Bulk modulus and compressibility of Incompressible fluid iv) Viscosity. |  |
| b) | State the law of static fluid. What is Pressure head? Explain briefly with neat sketch. |  |
| c) | Express intensity of Pressure of 1kgf/cm2 in terms of metres of water and ‘cm’ of mercury. |  |
| d) | What is Orifice and Mouthpiece? What role it plays if connected to a tank containing water? What is meant by Venacontracta? |  |
| e) | State ‘Bernaullis Theorem’. What are its limitations? Give equation of Bernaullis when applied to real fluid. |  |
| f) | Compare Laminar and Turbulent flow. What is Reynolds number? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State ‘Pascal Law’. Find the depth below oil surface, where intensity of Pressure is 20.3 KPa. Specific gravity of oil is 0.9. |  |
| b) | A Differential manometer connected at two points A and B at the same level in a pipe containing oil of specific gravity 0.8, shows difference in mercury level at 90 mm. Draw neat sketch and determine difference in Pressure at the two points. |  |
| c) | With the help of neat sketch, explain Mechanical Pressure Gauges. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define Total Pressure and Center of Pressure for a circular plate immersed vertically in water. Draw neat sketch showing the position on the plate. |  |
| b) | A rectangular plate 1m wide and 2m deep is immersed in water with its plane making an angle of 30 with water surface and its upper edge 1.5m below the water surface. Calculate Total Pressure force and Centre of Pressure. |  |
| c) | Find Total Pressure force and Centre of Pressure exerted on side tank, which is 1.5m wide and 1m deep. The tank is half full with liquid having specific gravity 2, while remaining liquid is having specific gravity of 1. Draw neat sketch and Pressure diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note on different types of flows in a pipe. Sketch and explain Pilot Tube to measures velocity of liquid. |  |

………2/-

-2-

| b) | A 300mm x 150mm Venturimeter is provided in a vertical pipeline carrying oil of specific gravity 0.9. The flow being upwards. The difference in elevation of the throat section and the entrance section of the Veturimeter is 300mm. The difference in mercury level is 250mm. Calculate the discharge. Draw a neat sketch. |  |
| --- | --- | --- |
| c) | A rectangular orifice 1.5m wide and 1.2m deep is fitted in one side of the tank. The water level on one side of the orifice is 2m above the top edge of the orifice, while on the other side it is 0.4m below its top edge. Calculate discharge through orifice. Take cd=0.62. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain “Hydraulic Coefficients”. Find the discharge from 80mm diameter external mouthpiece fitted to a side of large vessel, when head over mouthpiece is 4m. |  |
| b) | Find the discharge through trapezoidal Notch, which is 1m wide at the top and 0.40m at the bottom and 30cm in height. The head of water on the notch is 0.2m. Take cd=0.62 (For Rectangular portion), cd=0.60 (For Triangular portion). |  |
| c) | Write a note on “Hydraulic Gradient Line” and “Total Energy Line”. Find the head lost due to friction in a pipe of diameter 300mm and length 50m, through which water is flowing at 3m/s using Chezy’s formula for which c=60. Take  (Kinematic viscosity)= 0.01 stoke. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note on “Hydraulic Ram” with a neat Sketch. |  |
| b) | Write a note on “Most Economical Section of Channels”. A rectangular channel carries water at a rate of 400 lit/sec when bed slope is 1 in 2000. Find the most economical dimension of the channel if c=50. |  |
| c) | Determine difference in the elevation or free surface level between water surfaces in the two tanks which are connected by a horizontal pipe of diameter 300mm and length 400m. Discharge rate is 300 lit/sec. Consider all loses in pipe. Take f=0.008. |  |

***********