## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Mechanical Engg./A/SB

Subject: Mechanical Engineering Materials(4031)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Distinguish between Elasticity and Plasticity. Explain with examples. |  |
| b) | Explain the properties and advantages of Grey cast iron. |  |
| c) | What is alloying of steels? Why it is done? |  |
| d) | “Hardening is always followed by tempering”- True or false. Give reasons. |  |
| e) | Discuss the various glass forming constituents. |  |
| f) | What is “Nylon”? Give its important properties and uses. |  |
| g) | List the advantages and disadvantages of Non-destructive testing. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | What are T.T.T. curves? Sketch the curves and state their importance as regards to heat treatment process. |  |
| b) | State the difference between - Ferrous and Non-Ferrous metals and - Brasses and Bronzes. Name the various alloys of Aluminium and mention their importance in Industries. |  |
| c) | Why steels need heat treatment? List the various heat treatment processes applied to steels. |  |
| Q.No.3. | Answer any three of the following: | 3 x 4 = 12 |
| a) | Give the properties and uses of H.S.S. |  |
| b) | Write brief notes on i)Gun metal ii)Babbit metal. |  |
| c) | What are free cutting steels? Give its properties and applications. |  |
| d) | State the effect of the following elements on the structure and properties of steel: i)Silicon ii)Tungsten iii)Nickel. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | What are abrasives? Classify them and state their types and applications in general. |  |
| b) | Explain in brief: - Properties of Refractories - Refractory Products. |  |
| c) | What do you understand by Polymers? Discuss the classification of Plastics & its engineering applications. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With reference to Rubber explain the following: - Neoprene Rubber - Silicon Rubber - Styrene Butadiene Rubber. |  |
| b) | Write a brief note on the following Heat insulating materials. - Asbestos - Thermocole - Glass wool & cork. |  |
| c) | Explain the Dye Penetrant Test carried out on materials with the help of a neat sketch. Give its applications & limitations. |  |
| Q.No.6. | Write short notes on any three of the following: | 3 x 4 = 12 |
| a) | Stiffness & fatigue of materials |  |
| b) | X-Ray Radiography Test |  |
| c) | PVC, Teflon & Polyethene |  |
| d) | Identification of materials based on ISI, AISI |  |
| d) | Metals for nuclear energy |  |

***********