## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Design of Structures(Steel)-I(4024)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS:800-1984 and Steel tables is permissible.

| Q.No.1. | Answer any three of the following: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Draw stress-strain curve for mild steel. Give its significance. |  |
| b) | Draw sketches showing types of butt-welds and fillet welds. State the situations when to adopt each type. |  |
| c) | List the various applications of steel structures. State the merits and demerits of steel structures. |  |
| d) | What is the effect of wind load on roofs? How will you consider it in the design of roof trusses? |  |
| e) | Draw sketches showing single and double lacing system. Also represent force in each lacing bar. |  |
| Q.No.2. | Answer the following: |  |
| a) | Two angles ISA 80 x 50 x 10 mm are on either side of a gusset plate 10 mm thick. Design the welded joint taking permissible stresses in angle and weld as 150 MPa and 108 MPa respectively. OR | (6) |
| b) | Two flat sections 90 x 10 mm and 90 x 8 mm are required to be connected by a welded joint. Use fillet weld. Permissible stresses in angle and weld are 150 MPa and 108 MPa respectively. | (6) |
| c) | What do you understand by the terms: - Throat thickness - Effective length - Overlap - Side fillet - End return. | (6) |
| Q.No.3. | Answer the following: |  |
| a) | Draw sketches of shapes used for compression members. | (4) |
| b) | Design a column section to support an axial load of 710 KN. The column is 7 m long and is effectively held in position at both the ends but restrained against rotation at one end only. Yield stress of steel = 250 MPa. ### OR | (8) |
| c) | Design a gussetted base for a column section consisting of ISHB 300 @ 630 N/m carrying an axial load of 1260 KN, resting on concrete pedestal. SBC of soil is 220 KN/m2. Safe bearing pressure of concrete = 0.25 fck. | (8) |
| Q.No.4. | Answer the following: |  |
| a) | Hall dimension – 14 m x 6 m R.C.C. slab - 12 mm thick; Unit weight 25 KN/m3 Rolled steel joists - 3 m(centre to centre) Floor finish above RCC slab – 3 cm; Unit weight 24 KN/m3 Live load on slab - 4.4 KN/m2 Bearing on both sides – 30 cms wall Design floor beam and apply check for deflection and shear. | (8) ……2/- |

-2-

| b) | What is plate girder? Under what circumstances you will use it? Give commonly adopted section. ### OR | (4) |
| --- | --- | --- |
| c) | What do you understand by the term laterally restrained beam? | (4) |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw sketches in the following cases: - Beam to Beam connection - Beam to Column connection. |  |
| b) | What do you mean by eccentric welded connection? Draw a sketch of welded connection with a force ‘W’ acting at an eccentricity ‘e’ in case of butt weld and fillet weld. |  |
| c) | A strut in a roof truss carries an axial load of 200 KN. Design a double angle section. Length of strut centre to centre of intersections is 2.15 m; yield stress of steel is 250 MPa. |  |
| d) | A tie member in a roof truss is to be designed to carry an axial pull of 130 KN. Design a suitable section for the member if the length of the member is 1.70 m. Also design a suitable welded connection. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the various types of loads to be considered (as per IS: 875) in the design of roof trusses? |  |
| b) | Draw a cross section showing the following parts: Roof covering, Purlin, Roof truss, Column cap, Column, baseplate. |  |
| c) | Verify wheather ISLB 150 @ 142 N/m is safe to be used as purlin. Span of truss 12.40 m Spacing of trusses 5.10 m Slope of truss 26º with horizontal Live load 220 N/m2 Wind load 1000 N/m2. |  |

***********