## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-II(4335)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is  compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use graph paper if required.

| Q.No.1. | Explain any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Capacity plan of a ship |  |
| b) | Dynamic stability |  |
| c) | Permeability and margine line |  |
| d) | Preparation to be made for inclining experiment |  |
| e) | Bonjean curves |  |
| f) | Stowage factor and broken stowage |  |
| Q.No.2. | Construct a curve of statical stability for M.V. Cargo– carrier when the displacement is 35000t and KG is 8m. From the curve find: | (12) |
| i) | The range of stability |  |
| ii) | The maximum GZ and heel at within it occurs |  |
| iii) | The angle of vanishing stability |  |
| iv) | Approximate – Gm. |  |
| Q.No.3. | Answer the following: |  |
| a) | Explain the procedure to obtain free board for a particular vessel. | (8) |
| b) | What is free board and why it is assigned to a ship? | (2) |
| c) | In comparison of a bulk carrier to an oil tanker of same size which ship will be assigned loss free board and why? | (2) |
| Q.No.4. | Answer the following: |  |
| a) | A box shaped vessel 150m long, 20m beam is floating upright in salt water at a draught of 6m F and A. The collision bulkhead is situated 8m from forward. Find the new draughts if the vessel is now bilged forward of the collision bulkhead. | (8) |
| b) | With neat sketch derive the expression to find the change in draught ford and aft when the trim “t” is known. | (4) |
| Q.No.5. | Answer the following: |  |
| a) | Briefly describe the procedure to carry out side launching. | (10) |
| b) | What is the purpose of carrying out pre-launching calculation? | (2) |
| Q.No.6. | A hull of a box-shaped vessel is 50m long and has a mass of 600 tonnes. The mass of the hull is uniformly distributed over its length. Machinery of 200 tonnes mass extends uniformly over the quarter length midship. Two holds extending over the ford and aft quarter length each having 141 tonnes of cargo stowed uniformly over their lengths. Draw the curve of shearing force and bending moments for this condition and state their maximum values. | (12) |

………..2/-

-2-

| Q.No.7. | Answer the following: |  |
| --- | --- | --- |
| a) | Explain with neat sketch the procedure to carry out bending moment calculation of a ship shaped vessel. | (8) |
| b) | With a neat sketch show how hogging and sagging occurs in a ship structure when a vessel is placed on to a typical wave profile. | (4) |
|  |  | …..3/- |