## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Analytical Instrumentation(5115)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Discuss Redox potential measurement and state its applications. |  |
| b) | Differentiate between Automated Analyzers and Automatic Clinical Analyzer. |  |
| c) | Elaborate on any three optical techniques used for continuous automatic analysis of materials. |  |
| d) | Sketch and explain the working and major advantages of ‘Back Scatter gage’. |  |
| e) | Explain the working of the liquid scintillation counter and state its applications. |  |
| f) | State the charge and penetrating power of radioactive radiations. |  |
| g) | Discuss the role of ‘Peak Picker’ in gas chromatography with the help of a sketch. |  |
| Q.2. | Answer the following: | 3 x 4 = 12 |
| a) | Discuss the range, working and applications of a flame photometer. |  |
| b) | State and explain Beer’s-Lambertz law for measurement of concentrations of liquid samples. Differentiate between UV Analyzers & IR Analyzers. |  |
| c) | Discuss the principle of working of Non-dispersive type IR spectrophotometry. Explain the working of this meter for a sample solution of three ingredients. |  |
| Q.3.A) | Answer any two of the following Questions: | 2 x 5 = 10 |
| a) | Define the term “Paramagnetism”. With the help of block diagram explain the working of deflection type paramagnetic oxygen analyzer. Discuss each block in brief. |  |
| b) | Discuss in brief the analyzer section of the Process Chromatography without any sketches: |  |
| c) | What is meant by “Calomel of Mercury”? Explain temperature compensation techniques for PH meter for acidic condition & alkaline condition. |  |
| B) | Explain only the principle for working of T.C.D. | (2) |
| Q.4. | Answer the following: | 3 x 4 = 12 |
| a) | State the sensor for moisture content measurement. Explain this measurement for online textile fabric. |  |
| b) | Explain the need and advantages of using radio-isotopic measurements over conventional methods. |  |
| c) | Discuss any four industrial applications of isotopes being used as tracers. |  |

………2/-

-2-

| Q.5.A) | Answer any two of the following Questions: | 2 x 5 = 10 |
| --- | --- | --- |
| a) | Without sketch explain various steps involved in sample analysis of C-H-N analyzer. |  |
| b) | Discuss continuous flow analyzers, elaborating on the heart of this analyzer. How computer can help this analysis? |  |
| c) | Discuss the various design features when an analyzer is selected for clinical analysis of a given problem. |  |
| B) | Sketch out the mixing procedure using a STAC cell. | (2) |
| Q.No.6. | Answer any three of the following: | 3 x 4 = 12 |
| a) | Design features for continuous copper strip measurement using Beta gauge. |  |
| b) | Explain On-line Potentiometric analyzers. |  |
| c) | Explain the techniques for managing the spare parts. |  |
| d) | Explain Liquid chromatography and detectors for the same. |  |

***********