## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-I(4001)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a neat figure of 30 m chain. |  |
| b) | Write down uses of surveying. |  |
| c) | Explain scales & representative fraction. |  |
| d) | What is direct ranging? Explain. |  |
| e) | Explain factors affecting selection of survey station. |  |
| f) | Define Quadrantal bearing, W.C.B. and Force bearing. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain method of stepping. |  |
| b) | Draw the conventional signs for: i)Railway double line ii)Wall & Gate iii)Huts iv)Jungle. |  |
| c) | Explain principle & working of optical square. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain field procedure of chain & cross-staff survey. |  |
| b) | Define & give examples for Fore bearing, Quadrantal bearing, Back bearing & whole circle bearing. |  |
| c) | The Fore bearings of the lines AB, BC, CD and DE are 45 30, 12015 , 21030 and 285 45 respectively. Find angles B, C and D. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Following are the bearings observed while traversing with a compass an area where local attraction was suspected. Find the correct bearings & also the true bearings if the magnetic declination is 4 west Line; F.B.; B.B.. AB; 59 00; 239 0. BC; 139 30; 317 0. CD; 215 15; 36 30. DE; 208 00; 29 00. EA; 318 30; 138 45. |  |
| b) | Explain Traversing by chain & compass. |  |
| c) | Define types of Bench marks and give examples for each. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain temporary & permanent adjustments of dumpy level. |  |
| b) | Explain profile levelling & cross-sectioning. |  |
| c) | The following consecutive readings were taken with a levelling instrument at intervals of 20m. Find the R.C. of all the points 2.375, 1.635, 0.610, 3.635, 2.845, 2.08, 1.830, 0.97, 0.435, 1.645, 2.250, 3.450 and 3.935m. The instrument was shifted after the fourth & eighth readings. The last reading was taken on a B.M. of RC 60.2 m. |  |

………2/-

-2-

| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Levelling staff |  |
| b) | Fly levelling |  |
| c) | Planimeter |  |
| d) | Box-sextant |  |

***********