## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electrical Engg./EI

Subject: Circuit Theory(4104)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Derive an expression for the power consumed in a R-C circuit. Draw the waveforms of voltage, current and power consumed. |  |
| b) | Derive the expression of transient voltage for a RC (DC) circuit. |  |
| c) | Find the voltage Vdc in the network shown in fig.1(a). |  |
| d) | Explain Resonance in series R-L-C circuit and derive expression for Resonant frequency. |  |
| e) | Define power factor and explain Active, Reactive. |  |
| f) | State Thevenin’s theorem and list down the steps to Thevenize a given circuit. |  |
| g) | List down the advantages and disadvantages of Hay’s bridge. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Apply Thevenin’s theorem to find the current flowing through 12-ohm resistor in the circuit shown in fig.2(a). |  |
| b) | Find the current in the 4-ohm resistance using the principle of superposition in the circuit shown in fig.2(b). |  |
| c) | Using nodal analysis, find the current in the various elements of the network shown in fig.2(c). |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A current of 5A flows through a non inductive resistance in series with a choking coil when supplied at 250V, 50Hz. If the voltage across the resistance is 125V and across the coil 200V, calculate - Impedance, reactance and resistance of the coil. - The power absorbed by the coil and - The total power. Refer fig.3(a). |  |

………2/-

-2-

| b) | A coil having a Resistance of 10 ohm and an inductance of 0.2H is connected to a 100V, 50Hz supply. Calculate i) The impedance of the coil ii) The reactance of the coil iii)The current taken and iv)The phase difference between current and the applied voltage. |  |
| --- | --- | --- |
| c) | Two Impedances given by Z1=10+j5 and Z2=8+j6 are joined in parallel and connected across a voltage of V=200+j0. Calculate the circuit current, its phase and the branch currents. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive an expression for half power bandwidth in a Resonating series RLC circuit. Write down expressions for w1 & w2 and using graphical representation show variation of impedance, capacitive reactance, inductive reactance, net reactance and current with frequency. |  |
| b) | A series R-L-C circuit consists of R=1000, L=100mH and C=10 picofarads. The applied voltage across the circuit is 100V. - Find the resonant frequency of the circuit. - Find the quality factor of the circuit at the resonant frequency. - At what angular frequencies do the half power points occur? - Calculate the bandwidth of the circuit. |  |
| c) | A coil of resistance 20 and induction 200 H is in parallel with a variable capacitor. This combination is in series with a resistor of 8000. The voltage of the supply is 200V at a frequency of 106 Hz. Calculate - The value of ‘C’ to give resonance - The Q of the coil - The current in each branch of the circuit at Resonance. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive relationship between line and phase quantities in a Delta connected system. Also give an expression for power in terms of line values. |  |
| b) | Three equal star connected inductors take 8 KW at a power factor 0.8 when connected across a 460V, 3 phase, 3 wire supply. Find the circuit constants of the load per phase. |  |
|  |  | …..3/- |
|  | -3- |  |
| c) | A Delta connected balanced 3 phase load is supplied from a 3 phase 400V supply. The line current is 20A and the power taken by the load is 10,000W. Find - Impendance in each branch. - The line current, power factor and power consumed if the same load is connected in star. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The four arms of a Hay’s ac bridge are arranged as follows. AB is a coil of unknown impedance, BC is a non reactive resistor of 1000. CD is a non reactive resistor of 833 in series with a standard capacitor of 0.38F. DA is a non reactive resistor of 16,800. If the supply frequency is 50 z, determine the inductance and resistance at the balance condition. |  |
| b) | In a 3 phase, 4 wire system, the current in the R, Y and B lines under abnormal conditions of loading were as follows. IR=100 30, IY=50300 IB=30180. Calculate the positive, negative and zero phase sequence currents in the R- line and the return current in the neutral conductor. |  |
| c) | A three phase 400V, 50HZ ac supply is feeding a 3 phase delta connected load with each phase having a resistance of 25, an inductance of 0.15 H and a capacitance of 120F in series. Determine the line current, Volt Ampere, active power and reactive power. |  |