## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Mechanical Engg.

Subject: Machine Drawing-II(4035)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Calculate the limit sizes for a shaft designated as 30f7. The following data is given: i) The fundamental deviation for shaft of size 30mm is –20 microns ii) The fundamental tolerance (IT) is 21 microns for shaft of grade 7. |  |
| b) | Indicate the roughness grade numbers and roughness symbols for the following Roughness grade values: i) 50 μm ii)25 μm iii)6.3 μm iv)0.8 μm. |  |
| c) | Interpret the meaning of geometrical tolerances of the parts shown in fig.1. |  |
| d) | Indicate with symbols each of the following: - Machining finish with planning to a surface roughness value of 3.2 microns - Machining allowance of 3mm - Sampling length of 12mm - Surface to be chrome plated. |  |
| e) | Draw the double line representation for the following pipe fittings. i) Coupling ii) 45 Elbow iii) Cap iv) Tee. |  |
| f) | Draw the sectional representation and the appropriate symbol for the following form of welds: - Single V-butt with flat finish - Square butt weld - Spot weld - Double V-butt weld. |  |
| Q.No.2. | Draw neat proportionate free hand sketches of any two of the following: | 2 x 6 = 12 |
| a) | Gib and cotter joint |  |
| b) | Unprotected type flange coupling (1/2 sectional FV and side view) |  |
| c) | Step cone pulley for V-belt |  |
|  |  | ….2/- |
|  | -2- |  |
| Q.No.3. | Answer any one of the following: | (12) |
| a) | Fig. No. 2 shows the details of a “Universal Coupling”. Draw the following views of the assembly according to the third angle method of projection: - Half sectional Front view - Left hand side view. |  |
| b) | Fig No.3 shows the assembly of “Vertical stuffing Box”. Draw the following: - Stuffing box body (Front and Top view) - Gland (Front and Top view) - Neck Bush (Front and Top view) - Nut and stud. |  |
| Q.No.4. | Fig.No.4 shows two orthographic views of the Jig plate of a drill jig. Redraw the views and incorporate the following details:- - Bottom surface of the jig plate to be indicated as datum ‘A’. - To face to be parallel to datum ‘A’ within a tolerance of 0.02mm - Tolerance of g6 on thickness of 25mm - Tolerance of H7 on  54mm - Roughness on the inner surface of hole of 54mm to be indicated by . - Radial run out of 0.1mm on the cylindrical surface of  216mm. | (12) |
| Q.No.5. | Answer the following: | 2 x 6 = 12 |
| a) | Fig.No.5 shows a composite welding of a plate and studs weld. Represent the following on the given views to join - Plate 1 to 2 – fillet weld all round - 1 to 3- single- U butt weld on site - 1 to 4 – Double-U butt weld on site finished flush at the top - 3 to 5-continuous fillet weld on the front side and intermittent fillet weld on the rear side starting with a weld length of 20mm followed by a gap of 30mm. |  |
| b) | Fig.No.6 shows a piping layout in the form of a block diagram with the names of various fittings mentioned in the blocks. Draw single line diagram for the same. |  |

………3/-

-3-

| Q.No.6. | Study the working drawing of a slip bush shown in fig No.7 and answer any six of the following: | 6 x 2 = 12 |
| --- | --- | --- |
| a) | What is the size of the hole in the bush? |  |
| b) | What is the meaning of 3x45? |  |
| c) | What is the depth of the drilled hole at the side of the bush? |  |
| d) | What is the tolerance provided on hole of 16mm? |  |
| e) | What is the method used for imparting a rough surface to the bush for the purpose of gripping? |  |
| f) | What is the roughness of the inner surface of the hole of 16mm? |  |
| g) | What is the size of the bar stock required for the manufacture of the bush? | Cont.. |