## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Production Engg.

Subject: Machine Tools & Production Process-III(4075)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Distinguish between wet and dry grinding processes. |  |
| b) | With respect to honing, answer the following: - Amount of material removed? - Materials which can be machined? - Uses? |  |
| c) | Write the steps involved in cutting threads on tapered surfaces. |  |
| d) | Enlist 3 merits and demerits of broaching. |  |
| e) | A large number of attachments are available for use on a tool and cutter grinders. Justify their uses for at least three attachments. |  |
| f) | Describe with neat sketches the “work stops” or “bar stops” used in turret lathes. |  |
| g) | Write any three uses of Automatic lathes. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the process of making a hexagonal bolt on a turret lathe. |  |
| b) | Describe the features of turret lathe headstock. Also list the different types of headstocks. |  |
| c) | Describe any one capstan and turret lathe mechanism. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give broad classification of Automatic and semi automatic lathes. Give brief description of each class. |  |
| b) | What is a Swiss type automatic screw machine? How it functions and what are its main applications? |  |
| c) | Write a note on bond materials used in grinding wheel manufacture. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With respect to Grinding wheels explain the terms: Grit, Grade and structure. |  |
| b) | Explain the processes of external and internal cylindrical grinding. |  |
| c) | With a neat sketch explain the working and main parts of a Universal Tool and Cutter grinder. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What are the different types of broaches used in Industries? Elaborate. |  |
| b) | Work out the cutting length and the number of teeth of a broach to be used in cutting a slot 8mm wide, 3mm deep and 80mm long. |  |
| c) | Explain with respect to thread cutting the following terms: (i) Pitch (ii) Minor diameter (iii) Pitch diameter (iv) Depth of thread (v) Major diameter (vi) Thread angle. |  |
| Q.No.6. | Write short note on any three: | 3 x 4 = 12 |
| a) | Cutting threads using die heads |  |
| b) | Principle of super finishing |  |
| c) | Hand lapping |  |
| d) | Balancing of grinding wheels |  |
| e) | Grinding of Tools used in machining |  |

***********