## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mining Engg.

Subject: Underground Mining Machinery(4294)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the composition of a wire rope? |  |
| b) | Write short notes on winding sheaves. |  |
| c) | Give the technical specifications of a coal drill. |  |
| d) | Write short notes on a road header. |  |
| e) | What are the different transporting systems used in underground mines? |  |
| f) | How does a drop warrick operate? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the types of wire ropes used in mines? Explain a round stranded rope. |  |
| b) | What are the parts of cage suspension gear? Give the purpose of each. |  |
| c) | Explain the differences in construction of run round and shunt back of pit layouts. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain any one type of drum profile. What safety features are used in mine winding system? |  |
| b) | Write short notes on (i) Koepe winding (ii) Guide ropes. |  |
| c) | An endless rope haulage operates in a road 800 m long having a gradient of 1 in 5 against the load. Full tubs carrying 1 T coal each and weighing 0.5 T are attached in pairs at intervals of 50 m and empty tubs are attached similarly at the other side. The rope weighs 3 Kg per metre and runs at 3 Kmph. Assuming tub friction as 1/40; rope friction as 1/20 and the gear efficiency as 80% ; calculate the BHP of motor required. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction & operation of a Tack hammer drill. |  |
| b) | Draw a schematic diagram of a Shearer & explain the parts. |  |
| c) | Explain the construction & operation of a LHD. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the direct rope haulage system. List down the safety devices used in the system. |  |
| b) | What are the different types of locomotives used in underground mines? Explain the flame trap & exhaust conditioner with neat sketch. |  |
| c) | A locomotive hauls a train of 16 mine cars weighing 1 T up an incline of 1 in 25 at a constant speed of 13.5 Kmph. If the locomotive weighs 6 T and rolling friction is 1/60, what must be the power of the locomotive? |  |
| Q.No.6. | Answer any three of the following: | 3 x 4 = 12 |
| a) | Briefly list down the steps involved in rope splicing. |  |
| b) | Write on any one type of mechanical brake used in winding. |  |
| c) | Write briefly on scraper loader. |  |
| d) | Explain the super elevation with regard to curves of track. |  |

***********