## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Marine Engineering-III(4342)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Dog clutch |  |
| b) | Band Brake |  |
| c) | Advantages of Hydraulic Power |  |
| d) | Quick closing valve |  |
| e) | Preferential Tripping |  |
| f) | MARPOL |  |
| g) | Mooring winch |  |
| h) | Three way cock or MCB |  |
| Q.No.2. | (i)Sketch and describe a lubricating oil system used for main engine. | (8) |
|  | (ii)How to know that the filter on the line has clogged and how it is dealt with? | (4) |
| Q.No.3. | (i)Sketch and describe steam system used in conjunction with Auxiliary Boiler-composite type. Why the boiler as well as the steam piping needs to be lagged? | (8) |
|  | (ii)What protection is provided to the system to avoid ‘water hammering’? | (4) |
| Q.No.4. | (i)Explain the terms ‘Ventilation’, ‘Air Conditioning’ and ‘Refrigeration’. What do you understand by the term ‘Number of Air changes and why it is necessary to have different Air changes for different compartments of the vessel? Which all places in ship may have ‘Natural Ventilation’? | (8) |
|  | (ii)Is the machinery common for ‘Refrigeration’ and ‘Air conditioning’? If not what are the differences? | (4) |
| Q.No.5. | (i)Describe the method or procedure of paralleling two A.C. Generators. Can three such generators be also run in parallel? What are essential fittings on main switchboard for paralleling operation and how do they differ from the fittings required for D.C. systems? | (8) |
|  | (ii)What is the purpose of ‘Preferential trips’ and are they necessary for A.C. system? What may happen if these trips are not incorporated on main switchboard? | (4) |
| Q.No.6. | (i)What equipment is used for handling ‘Sewage’ or ‘Garbage’ on board ship and how does that function? | (8) |
|  | (ii)Who is concerned or bothered when untreated sewage or garbage is disposed off at sea, when it is detected? | (4) |
| Q.No.7. | Why does a ship require ‘Compressed Air system’? What type of Air Compressors used on board ship? What are safety features on Compressed Air System? | (12) |

***********