## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-I(4037)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Steam Tables and Mollier’s Charts is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Describe i) Bio-gas energy ii) Hydraulic energy. |  |
| b) | Define calorific value of fuel. What is the difference between higher calorific value and lower calorific value? |  |
| c) | State first law of thermodynamics. What are the limitations of first law of thermodynamics? |  |
| d) | What is the difference between universal gas constant and characteristic gas constant? |  |
| e) | Define the terms (i) Latent heat of vaporization (ii) Dryness fraction. |  |
| f) | State three merits and three limitations of solar energy. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the Bomb calorimeter and the procedure to calculate Higher calorific value. |  |
| b) | A certain quantity of air has a volume of 0.03m3 at a pressure of 1.3 bar and 27C. It is compressed to a volume of 0.004m3 according to the law PV1.4 = Constant. Find the final temperature and work done. |  |
| c) | Define and represent the following processes on P-V and T-S diagram: i) Adiabatic process ii)ISO choric process. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the internal energy of 1Kg of steam at 20bar in the following cases: - When the steam is wet having dryness fraction 0.7. - When the steam is superheated to 400C. Take specific heat for superheated steam as 2.3 KJ/KgK. |  |
| b) | Show Rankine cycle on P-V and T-S diagrams and explain the processes involved. Also draw the mechanical system to show different processes of Rankine cycle. |  |
| c) | With a neat sketch explain the use of any steam calorimeter for measuring dryness fraction of steam. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the purpose of blow off cock? Explain its construction with the help of a neat sketch. |  |
| b) | Describe with a neat sketch the construction and working of a Babcock and Wilcox boiler. |  |
| c) | Explain with a neat sketch the construction and working of a La-Mont boiler. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What is the function of a superheater? Describe the construction of a superheater with the help of a neat sketch. |  |
| b) | Steam at a pressure of 15 bar and temperature of 300C is supplied to a steam turbine working on the Rankine cycle. If the exhaust takes place at 0.15 bar, calculate the Rankine efficiency. |  |
| c) | With the help of a neat sketch explain the construction and working of Axial flow compressor. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Uses of compressed air |  |
| b) | Inter-cooling in air compressors |  |
| c) | Vane type air compressor |  |
| d) | Boiler draught |  |
| e) | Gay-Lussac Law |  |

***********