## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Automobile Engg.

Subject: Design & Drawing of Auto Components(4067)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List any four primary causes of mechanical failure. |  |
| b) | What is a Key? List any 4 types of Keys. |  |
| c) | What is ‘spring index’ and ‘spring rate’? |  |
| d) | Differentiate between clutch and coupling. |  |
| e) | Explain the three types of fits. |  |
| f) | A M.S bolt is subjected to an axial load of 50 KN. Determine the size of the bolt if permissible tensile stress for bolt material is 100 MPa. Consider tension failure across the core section of the screw. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The screw of a shaft straightener as shown in fig.1 is subjected to a maximum load of 20 KN. Determine the size of the arm section, if b=3t for the material of the arm. The allowable stresses in compression and tension are 60 MPa and 50 MPa respectively. |  |
| b) | A mild steel shaft as shown in fig.2 receives 25KW power at 500rpm through a belt pulley of diameter 500 mm. Ratio of tension of the belt is 3:1 and weight of the pulley is 500N. Determine the diameter of the shaft if permissible shear stress for the material is 40 MPa. |  |
| c) | Fig.3 shows a bolted joint in which the bolt is subjected to an axial load of 20 KN. Determine the dimensions mentioned in the figure for the following stresses of the bolt material. t = 80MPa , s = 60MPa, cr = 120MPa |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cast iron bracket as shown in fig.4 supports a load of 10 KN. It is fixed to a horizontal channel by means of 4 identical bolts, two at A and two at B. The bolts are made of steel 30C8 (Syt=400 N/mm2) and the factor of safety 6. Determine the size of the bolts. | ……2/- |
|  | -2- |  |
| b) | A three hole bracket as shown in fig.5 is fixed to a wall by 3 bolts. Find the size of the bolts considering a load of 12 KN to act at 200mm from the wall. Take allowable tensile and shear stresses for the bolts as 42 MPa and 28 MPa respectively. |  |
| c) | A rigid coupling is used to transmit 25 KW at 720 rpm. There are four bolts and P.C.D. of the bolts is 125 mm. The bolts are made of steel 45C8 (Syt= 380 N/mm2) and factor of safety is 3. Determine the diameter of the bolts. Take Sys= 0.577 Syt. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A shaft 80 mm diameter transmits power at maximum torsional shear stress of 63 N/mm2. Find the length of 20 mm wide sunk key required to mount a pulley on the shaft so that the maximum shear stress in the key does not exceed 42 N/mm2. |  |
| b) | Determine the diameter of hollow push rod made of mild steel of an I.C. engine if maximum force exerted by push rod is 1500 N. The length of the push rod is 50 cm. Factor of safety = 2.5, E= 210 KN/mm2. |  |
| c) | A multiplate disc clutch employs 3 steel and 2 bronze discs having outer diameter 300 mm and inner diameter 200 mm. Coefficient of friction is 0.22. Find the axial pressure and power transmitted at 720 rpm if normal unit pressure is 0.13 N/mm2. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a helical compression spring for a maximum load of 1000N for a deflection of 25mm using the value of spring index as 5. The maximum permissible shear stress for spring wire is 420 N/mm2. The modulus of rigidity is 84 N/mm2 and the modulus of elasticity is 200 N/mm2. |  |
| b) | Design a spring using the following data: Total load=140 KN, Number of springs supporting load= 4, Maximum number of leaves = 10, span of the spring = 1000 mm, permissible deflection = 80 mm, Young’s modulus E= 200 KN/mm2, allowable stress in spring material = 600 N/mm2. |  |
| c) | Determine the dimensions of an I- section of a connecting rod for an I.C. engine having the following specifications: Diameter of piston = 120 mm; Stroke = 130 mm, Weight of reciprocating parts = 3000 N/m2 of piston area, Length of connecting rod = 350 mm, R.P.M. = 1800, Maximum explosion pressure = 3 MPa, ultimate crushing stress for the material = 330 N/ mm2. Take Factor of safety = 6. |  |

………3/-

-3-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | - A single plate clutch effective on both sides is engaged by means of 6 helical springs. The coefficient of friction is 0.3 and the mean diameter of friction surface is 228 mm. Find the axial force exerted by each spring if the torque transmitted by the clutch is 245 N-m. - List the different clutches. Also list the clutch lining materials. | (03) (03) |
| b) | Design the crown thickness of a C.I. piston for a single acting 4 stroke engine for the following specifications: Cylinder bore = 102 mm; Stroke = 120mm Max. Gas pressure = 4.8 MPa; b.m.e.p = 0.6 MPa Speed = 2200 rpm; Fuel consumption = 225 gm/Kw-hr Allowable tensile stress for CI = 37 MPa Temperature difference between centre and edge of crown = 220C Heat conductivity factor = 47 w/m C H.C.V. of fuel = 41880 KJ/Kg. | (06) |
| c) | Draw a stress strain diagram and explain the following: - Proportional limit - Yield strength - Ultimate strength | (1½) (1½) (1½) (1½) |