## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Process Control(4203)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the symbol of control relay, solenoid and level limit switch. |  |
| b) | With the help of an example explain i)cycling ii)dead time. |  |
| c) | Explain in brief the elements in any process control system with block diagram. |  |
| d) | Sketch the constructional details of a control valve and name its parts. |  |
| e) | What are the loop disturbances that occur in a process control system? |  |
| f) | List the methods of process loop tuning and explain one method. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a wiring diagram explain the input, output modules connention to a PLC. |  |
| b) | With the help of an example explain ladder diagram elements and concept of ladder diagram. |  |
| c) | What are relay sequencers? Compare their performance with PLC. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of process equation and graph explain three position controller. |  |
| b) | Explain composite controller modes and their features. |  |
| c) | Explain the terms process load, process lag and self- regulation. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of graphs explain single speed floating controller and derivative mode controller. |  |
| b) | Explain the implementation of proportional Integral Derivative Controller mode using op-amp with the help of a circuit diagram. |  |
| c) | Draw the schematic diagram and explain the implementation of Integral mode using pneumatics. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain working of a data logger using computer with help of a block diagram. |  |
| b) | Explain Direct Digital control with the help of a block diagram. |  |
| c) | Write a short notes on: i) Multi variable control ii) Cascade control. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Give example of two mechanical control elements and explain the working. |  |
| b) | Explain any three signal conversion techniques. |  |
| c) | With the help of examples compare hydraulic and electrical actuators. |  |

***********