## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mechanical Engg./A

Subject: Machine Drawing-I(4033)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Attempt any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Fig.1 shows two views of a machine component drawn by third angle method of projection. Draw the auxiliary top view in the direction of arrow ‘X’. |  |
| b) | A rectangular hexagon, of 2.5 mm side, rests on one of its sides in HP. Its plane is inclined to the HP at 45 and the side, on which it rests, inclined at 30 to the V.P. Draw its projections using auxiliary plane method. |  |
| c) | A right circular cylinder, diameter of base 40 mm and height 60mm, is resting in HP on its base rim such that its axis is parallel to V.P., inclined at 30 to the HP and is 40 mm away from the VP. Draw its projections by change of ground line method. |  |
| Q.No.2. | A pentagonal pyramid, side of base 30mm and height 52 mm, stands with its base on HP and an edge of the base is parallel to V.P. It is cut by a plane perpendicular to V.P., inclined at 40 to HP and passing through a point on the axis, 32 mm above the base. Develop the lateral surface of the truncated pyramid. | (10) |
|  | #### OR |  |
| Q.No.2. | Develop the inside pattern of the right circular cylinder cut at top and bottom, as shown in fig.No.2. | (10) |
| Q.No.3. | A vertical cylinder of 70 mm diameter is penetrated by another cylinder of the same size. The axis of the penetrating cylinder is parallel to both the HP & VP and is 9 mm away from the axis of the vertical cylinder. Draw the projection showing the curves of intersection. | (12) |
|  | #### OR |  |
| Q.No.3. | A vertical cylinder of 60 mm diameter is penetrated by a horizontal square prism, base 40 mm side, the axis of which is parallel to the VP and 10 mm away from the axis of the cylinder. A face of the prism makes an angle of 30 with the HP. Draw their projection showing curves of intersection. | (12) |
| Q.No.4. | Fig.3 shows by 1st angle method of projection the front view and top view of an object. Draw using the same angle of projection: | (16) |
|  | i) Top View ii) Sectional front view along A-A |  |
|  | iii)Left hand side View. |  |
| Q.No.5.a) | ### With the help of a suitable free hand sketch show the following types of sections: | (08) |
|  | i) Spoked wheels section ii)Partial section |  |
|  | iii)Aligned section iv)Offset section. |  |
|  | #### OR | ….2/- |

-2-

| a) | Give conventional representation for the following: | (08) |
| --- | --- | --- |
|  | i) Glass Packing ii)Insulation Material |  |
|  | iii)Bush bearing iv)Springs. |  |
| b) | Draw by single line method i) Gate valve ii) Cap iii) Cross. | (09) |
| Q.No.6. | Fig.4 shows the view of a machine part to be fabricated by the welding process. Select suitable types of welds and indicate the same through the appropriate symbols. | (08) |