## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Civil Engg./CT

Subject: Analysis of Structures(4026)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data, if required, and state it clearly.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | What are shear connectors? Explain why they are necessary. How they are designed? |  |
| b) | Derive an expression for fixed end moments for a fixed beam of span L carrying an udl of w KN/m over the entire length. |  |
| c) | A point in the body is subjected to axial tensile stress of 70 N/mm2. Calculate normal and tangential stresses on a plane inclined at 40 with the horizontal axis. |  |
| d) | Derive an expression for eccentricity for a circular column carrying an eccentric load, for no tension condition. |  |
| e) | State assumptions made in Eulers theory for columns. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel plate of 15 mm thickness and 200mm depth is symmetrically placed between two timber joists, each of which is 150mm wide and 250mm deep. Determine moment of resistance of the flitched beam thus formed if stress in steel and timber are not to exceed 110N/mm2 and 7N/mm2 respectively. Take Es =17 Etimber. |  |
| b) | A short circular hollow column has external diameter of 300 mm and internal diameter of 240mm. It is subjected to axial load of 300 KN and an eccentric load of 80KN acting at a distance of 500mm from axis of the column, acting on a bracket attached. Find maximum and minimum stresses induced in the column section. |  |
| c) | Explain with neat sketch: (i) Core section (ii) transformed section (iii) limit of eccentricity (iv) composite section. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Principal stresses acting at a point in the section of the member are 180N/mm2 and 100N/mm2, both tensile. Using Mohr’s circle method locate the plane across which resultant has maximum obliquity. |  |
| b) | At a point in strained material, the normal stresses on two planes, mutually perpendicular, are 65N/mm2 and 40N/mm2 both compressive. In addition to the above a shear stress of 30 N/mm2 is also acting on these planes. Calculate maximum shear stress and locate the plane for which the resultant has maximum obliquity. |  |
| c) | Principal stresses at a point in the material are 150 N/mm2 tensile and 75 N/mm2 compressive. Determine normal, tangential and resultant stresses on a plane, inclined at an angle of 50 to the axis of major stress. |  |

………2/-

-2-

| Q.No.4. | Answer any one of the following Questions: | (12) |
| --- | --- | --- |
| a) | A cantilever beam 300mm wide and 400 mm deep carries an udl of 20 KN/m over its entire length of 4m. In addition to this, it carries two point loads of 25KN and 50 KN at centre and at free end respectively. Determine slope and deflection under the load of 25KN and at the free end. Take E=0.3 x 105 N/mm2. Use Macaulay’s method. |  |
| b) | A fixed beam of 8 m span carries an udl of 25KN/m over its entire length. In addition, the beam carries two point loads of 35KN and 50KN acting at a distance of 3m and 6m from left support respectively. Determine reactions at the supports A & B and draw shear force and bending moment diagram. |  |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | A portal frame ABCD with supports A and D, both fixed carries an udl over beam BC of 30KN/m. Beam BC also carries a point load of 45KN at the centre. Using moment distribution method calculate moments and plot bending moment diagram for the frame if columns AB and CD are of length 4m each and has flexural rigidity of 2I. Beam BC is 5 m long with flexural rigidity of 1.5EI. |  |
| b) | A two span continuous beam ABC has support A fixed and support C simply supported. Beam AB is 4.5m long & carries udl of 35KN/m and has flexural rigidity of 1.5I. Beam BC has length of 5m and carries udl of 25KN/m and a concentrated load of 50KN at centre. It has flexural rigidity of 2 EI. Using moment distribution method, calculate reactions and bending moments and plot SFD & BMD. |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Compute Rankines critical load for a hollow circular column having internal diameter of 250mm and external diameter of 300mm. Column is 6.5 m long with one end fixed and other end hinged. Take fc=400N/mm2 and Rankines constant = 1/750. |  |
| b) | Find the ratio of buckling load resisted by the hollow and solid steel columns having same lengths and same cross sectional area. Assume both ends fixed and take external diameter = 1.5 internal diameter. |  |

***********