## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Architectural Engg.

Subject: Building Services-II(4533)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain Acoustics. |  |
| b) | What is the function of the Evaporator in the A/C unit? |  |
| c) | What is meant by Mechanical Ventilation. |  |
| d) | What are combustible materials? Mention any two of the combustible materials. |  |
| e) | Explain the meaning of thermal insulation. |  |
| f) | What is air conditioning? List its purpose in the various area of use. |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Name and describe any two methods of Mechanical Ventilation. |  |
| b) | What do you mean by humidification and dehumidification. Briefly explain why and how each of the devices plays its role in air conditioning system. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In a high-rise commercial building, what type of fire alarm system would you recommend? Give reasons and elaborate. |  |
| b) | Compare the fire resisting properties of steel with that of R.C.C. slab. In view if this, what would you suggest in a fire resistant construction, a steel column or a R.C.C column. |  |
| c) | List down various fire extinguishing equipments you would use in a high rise building. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the thermal insulation adopted for sloping roofs, brick walls and floors. Name any two types of thermal insulation material, which are used, for wooden roofs. |  |
| b) | What do you understand by reverberation? Define reverberation time. Briefly comment on the optimum time of reverberation in a Cinema theatre as compared to that of a Lecture Hall. |  |
| c) | Explain and draw different sketches to show thermal insulation adopted for an Industrial building. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write short notes on: i) Humidification ii) Exhaust system of Ventilation. |  |
| b) | What are the requirements of a good Ventilating system? Give any 6 good points. |  |
| c) | Differentiate between Central air conditioning system and Split air conditioning system. |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Mention the precautionary measures, which can be taken to avoid or minimize dangers of fire. |  |
| b) | “Different means of fire escape”- Explain with the help of neat sketches of any high rise building. |  |

***********