## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Drives(4115)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the factors on which selection of motor of a drive depend. |  |
| b) | Classify the loads to be driven by electric motor. |  |
| c) | State the factors on which variation of load torque depends, stating one example of each. |  |
| d) | Define stability of a drive system. Also state the types of stabilities. |  |
| e) | Which of the electrical braking is most efficient and why? |  |
| f) | State the type of motor used for following drives and justify your choice: i) Centrifugal pump ii) Lifts. |  |
| g) | State the types of motors used in paper mill. |  |
| h) | Give two points of distinction between relay & electromagnetic contactor. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a four quadrant diagram to indicate the operation of a hoist and name each quadrant operation. Also indicate the directions of motor torque, Load torque and speed. Also draw speed v/s torque characteristics on it. |  |
| b) | Define i) Equivalent Torque ii) Equivalent moment of Inertia of a drive system. Also state the expression for each, indicating each term used in it. |  |
| c) | 1)Draw speed v/s torque characteristics for i)Dry friction load ii) Viscous friction load iii)Fan type load. 2)State the equation relating motor torque, load torque and inertia torque in a drive system. Also state the effect of following conditions on drive system: i)Motor torque > Load torque ii)Motor torque < Load torque. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat circuit diagram, explain the method of rheostatic braking in d.c. shunt motor. Also draw speed v/s torque characteristic for rheostatic braking in d.c. shunt motor. |  |
| b) | Draw a neat circuit diagram for rheostatic braking in d.c. series motor using any one method of your choice and explain it in brief. Also draw speed v/s torque characteristics for such braking. |  |
| c) | With the help of neat circuit diagram, explain in brief plugging in three phase Induction motor. Also draw its speed v/s torque characteristic. |  |

………2/-

-2-

| Q.No.4. | Answer any three of the following Questions: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Explain the constant torque operation in using pole changing method for speed control of 3 phase Induction motor with the help of circuit diagram and speed torque characteristic. |  |
| b) | State the methods used to achieve rotor resistance control. How can it be achieved using thyristorised device? Explain it in brief with the help of neat circuit diagram. |  |
| c) | State the effect of cascade connections on Inductions motor. Hence explain this method in brief with the help if neat circuit diagram. |  |
| d) | Explain in brief the use of Stator Voltage Control in speed control of 3 phase Induction motor with the help of neat circuit diagram. |  |
| e) | State the different types of enclosures used for electric motor. |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | State the methods of determining power rating of motor with continuous duty & variable load and explain each in brief. |  |
| b) | Explain in brief the effect of environmental factors on determination of rating of motor. |  |
| c) | State the causes of load fluctuations. What do you understand by load equalization? |  |
| d) | Explain the working of float switch with the help of neat diagrams. |  |
| e) | Explain in brief the working of overload relay with the help of neat diagram. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the necessity of sequential control operation in an electric motor? Explain the control circuit operation of sequential control using Time delay with the help of neat control circuit diagram. |  |
| b) | State the methods of reversing of direction of rotation of three phase Induction motor. Explain any one such method of your choice with the help of its control circuit diagram. |  |
| c) | Draw the control circuit diagram & explain in brief remote control operation of an induction motor. |  |

***********