## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Civil Engg./CT

Subject: Public Health Engineering(4015)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Importance of protecting Environment |  |
| b) | Ground water and surface water sources |  |
| c) | Sanitation, hygiene and pollution other than water supply |  |
| d) | Factors controlling selection of sources of water |  |
| e) | Continuous and intermittent water supply systems |  |
| f) | Losses of water in a water supply system |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Enlist any three physical characteristics, three chemical characteristics and one biological characteristics of water. Indicate also the safe permissible limit of each. |  |
| b) | What is the necessity of studying variation in demand? Show with a neat sketch, hourly variation of water demand for an Indian town. |  |
| c) | Explain with a sketch the simple method of finding turbidity of water. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat flow diagram of a typical water treatment plant and label all the units with flow direction. |  |
| b) | What are the four principles of filtration of water? Explain each in brief. |  |
| c) | Draw neat sketches and explain the three methods of distribution of water pressure lines, depending upon the method of developing pressure. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch showing ferrul joint connection of water supply to a building including inlet pipe to overhead tank and all pipelines to different parts of a single storeyed building. |  |
| b) | Define: Sullage, domestic sewage, sewerage, sewer, dry weather flow, outfall sewer. |  |
| c) | Write any six advantages of water carriage system over conservancy system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw neat two alternate flow diagrams of sewage treatment plant for different types of secondary treatment. Label all the units of treatment. |  |
| b) | Draw and explain working of trickling filter. |  |
| c) | Write on methods of disposal of effluent. |  |

………2/-

-2-

| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Write a short note on methods of disposal of dry refuse. |  |
| b) | Draw neat sketches showing single stack system, one pipe system and two pipe system of house plumbing. |  |
| c) | Define dry refuse, garbage, rubbish. |  |
| d) | Draw neat sketch of a septic tank and soak pit (Cross section). |  |
| e) | Write short notes on functions of traps, types of traps in house drainage system. |  |

***********