## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Common

Subject: Elements of Mechanical Engineering(3004)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a simple gear train and write down the expression for its train value. |  |
| b) | Why does a diesel engine require heavier flywheel as compared to a petrol engine? |  |
| c) | Write down the classification of boilers. |  |
| d) | List down any six important applications of the compressed air. |  |
| e) | State any three important functions performed by a lubrication system. |  |
| f) | What is priming? Why is it required in the centrifugal pump? |  |
| g) | Differentiate between a roller bearing and a journal bearing. (Give any three important points) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Two pulleys having diameters of 340 mm and 280 mm and 2.4 m as the centre to centre distance, are to be connected using belt. Calculate the length of belt when, 1)Both pulleys are to rotate in same sense, using open belt system. 2)Both pulleys are to rotate in opposite sense, using cross belt system. Assume that the pulleys are mounted on the parallel shafts and neglect the thickness of belt. |  |
| b) | Write a note on the material used for manufacturing belts. |  |
| c) | List down the methods of lubrication and discuss any one of the methods giving neat sketch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain the working of two stoke cycle petrol engine. |  |
| b) | Giving neat sketch, explain the working of a simple carburettor. |  |
| c) | Write a note on air motors. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Giving neat sketch explain the construction and working of steam boiler. |  |
| b) | With the help of neat sketch explain the working of a reciprocating pump. |  |
| c) | Enlist any six troubles and remedies with respect to the hydraulic pumps. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Giving neat sketch explain the working of a rotary compressor. |  |
| b) | Differentiate between two stroke and four stroke cycle engine. (Give any four points of differences) |  |
| c) | List down the various boiler accessories and explain any one of the boiler accessories giving neat sketch. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Sliding contact bearings |  |
| b) | Pressurised water cooling system |  |
| c) | Principle of working of pneumatic tools |  |
| d) | Bernoulli’s theorem |  |
| e) | Head losses in pipes |  |

***********