## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electrical Engg./E&EE

Subject: Utilisation of Electrical Energy(4111)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following terms with respect to illumination: i) MHCP ii) Absorption factor. |  |
| b) | Explain the classification of systems of electric traction. |  |
| c) | List the properties a good heating element should possess. |  |
| d) | List any six methods of Electric heating. |  |
| e) | Explain with a neat diagram Spot welding. |  |
| f) | Explain Infra red heating and give their applications. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain modes of heat transfer and give the expressions for heat transfer for each mode. |  |
| b) | With a neat diagram explain the following methods of heat control: i) using selector switch ii) using tapped transformer. |  |
| c) | Explain any three causes of failure of heating element and explain effect of supply voltage variation on heat generated by heating element. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare the Resistance and arc welding. |  |
| b) | Explain Eddy current heating. Give its application. |  |
| c) | Explain phenomenon of Arc blow and list down methods for reducing it. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain principle of Arc welding? State and explain briefly the requirements of Arc welding source. |  |
| b) | With a neat diagram explain the various current collecting devices used in overhead supply for electric traction. |  |
| c) | Explain different speed control methods employed for traction motors. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define and explain the following terms: i) Luminous flux ii) Lumen iii) Candle power. |  |
| b) | With a neat diagram explain the construction and working of a High pressure sodium vapour lamp. |  |
| c) | Write short note on: i) Colour rendering ii) Stroboscopic effect. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you understand by speed time curves? Draw the speed time curves for urban and main line service. |  |
| b) | Explain glare and shadows. Also list down the remedies for the same. |  |
| c) | Briefly explain Dielectric heating and list down its application. |  |

***********