## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mechanical Engg.

Subject: Metrology & Quality Control(4047)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is precision? How does it differ from accuracy? |  |
| b) | With the help of a neat diagram derive a mathematical expression to prove that sine bar is not recommended for angles larges than 45 with the reference plane. |  |
| c) | Differentiate between Shaft based system and Hole based system. |  |
| d) | Explain the symbols used for designating the surface roughness on drawing. |  |
| e) | What are Drunken threads? |  |
| f) | What are the objectives of quality control? |  |
| g) | What is Budgeting? |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain how a Clinometer is used to find the inclined angle between two adjacent faces of a workpiece. |  |
| b) | Find the limit dimensions of a clearance fit used on the hole basis system for a basic size of 30mm diameter with a minimum clearance of 0.15mm. Tolerance of hole is 0.035 mm and that of shaft is 0.045mm. |  |
| Q.No.3. | Answer any two the following Questions: | 2 x 6 = 12 |
| a) | Compare Pneumatic and mechanical comparators. What are the advantages of optical comparators? |  |
| b) | Explain the following in the content of surface finish: |  |
|  | i)Root mean square value ii)Centre line average value. |  |
| c) | What are the sources of errors in manufacturing gears? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the significance of Quality of Design and Quality of Conformance. |  |
| b) | How can cost and value of quality be measured and balanced? |  |
| c) | Explain the following: |  |
|  | i)Quality audit system ii) Vendor Quality rating. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the difference between control chart for variables and control chart for attributes. |  |
| b) | Explain single sampling plan and double sampling plan with respect to their respective acceptance criteria. |  |
| c) | Following details are available regarding certain dimension of a component for which X- chart is to be prepared: |  |
|  | i)The sub group size is 4 |  |
|  | ii)After 20 subgroup it is found that ΣX=900mm and ΣR=6.22mm. |  |
|  | iii)d2 for sub-group size 4 is 2.059. Compute the centre line and the control limits for X- chart. | ………2/- |

-2-

| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| --- | --- | --- |
| a) | Need of inspection in industries |  |
| b) | Requirements of good dial indicator |  |
| c) | Surface texture |  |
| d) | Specification of quality |  |
| e) | Process Capability |  |
| f) | Operating Characteristic curve |  |

***********