## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mining Engg.

Subject: Surface Mining(4298)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define stripping ratio and enumerate various types of it. |  |
| b) | Write in brief on inclined hole blasting. |  |
| c) | Write on Cycle time of excavator. |  |
| d) | Write in brief on Haul road construction material. |  |
| e) | Write in brief on Selection of dumping site. |  |
| f) | Write in brief on maximum permissible bench slope and bench height as per regulation. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain methods of ripping. |  |
| b) | Write in brief on Types of drilling methods used in open cast mining and their suitability conditions. |  |
| c) | Write on Empirical rules for blast design. |  |
| d) | Write on Selection of explosive. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain extraction methods of dragline. |  |
| b) | Explain over casting by power shovel. |  |
| c) | Write on working near mine boundary. |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | Explain methods of haul road construction. | (5) |
| b) | The following information is provided for overburden preparation requirements of surface mining: Spacing of drill hole – 7m Burden of drill hole – 4m Over burden to be removed per month – 4,40,000m3 Shift/day – 2 Hours of working per shift –6 Drilling days per week –5 Drilling rate –20m/hour Find the total No. of drill required. ### OR | (7) |
| b) | A shovel of 4.6 m3 capacity is employed in a project and particulars are as follows. No. of pass : 5 Bulk density of material : 1.7 t/m3 Bucket fill factor – 75% Haul distance –2.5Km Speed of dumper – 20Km/hr Shovel cycle time – 25 seconds. Calculate 1) hourly capacity of shovel 2) capacity and number of dumpers. | (7) |

………2/-

-2-

| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain various types of mine planning. |  |
| b) | - Classification of open cast mining method. - Explosive used in open cast mining. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Suitability conditions of dragline |  |
| b) | Transport system in open cast mining |  |
| c) | Dump scope stabilization |  |
| d) | Methods of land reclamation |  |

***********