## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Civil Engg./CT

Subject: Design of Structures-II(RCC)[4016]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS456-2000 code is permissible.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Explain what do you understand by ‘under reinforced’, ‘Balanced’ and ‘over reinforced’ sections. Stating the reasons, which section do you prefer? |  |
| b) | Define ‘Limit state’. Explain various Limit states considered in design of R.C. members. |  |
| c) | - List out various loads and their combinations considered in the design of R.C. members. - Explain the reason why Indian standard permits higher partial safety factor for concrete than steel reinforcement. |  |
| d) | What is ‘Structural Integrity’? Explain how do you establish the same in your design. |  |
| e) | Sketch various types of cover blocks used for different structural members like slabs, beams and columns. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An R.C. beam 230mm x 450mm (overall) is reinforced with 3#16mm as tensile reinforcement. If M20 concrete and Fe415 steel are used for the beam. Determine the safe Udl the beam can support over the simply supported span of 4.5m at working condition. |  |
| b) | Design an R.C. beam of rectangular section to carry an ultimate load of 40 KN/m over an effective span of 5m. M25 concrete and Fe500 steel are used for beam section. |  |
| c) | A simply supported beam 250mm x 400mm (effective) is reinforced with 4#12mm. Taking M25 concrete and Fe415 steel calculate depth of N.A., lever arm and check for minimum reinforcement as per IS code. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported beam 230mm x 450mm (effective) carries an udl of 35KN/m under working condition over an effective span of 5m. The beam is provided with 4#12mm. Design the shear reinforcement for the beam, if M25 concrete and Fe 415 steel are used. |  |
| b) | Explain the terms flexural bond and anchorage bond. What is the development length Ld, if  is the diameter of bar with Fe415 steel and M25 concrete for bar in tension. |  |
| c) | A simply supported beam of rectangular section has to carry the superimposed load of 30 KN/m under working condition over an effective span of 4.75m. Design the beam with suitable shear reinforcement. M25 concrete and Fe415 steel are available for the beam. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Design a rectangular beam, whose dimension is restricted to maximum of 250mm x 400mm to carry the superimposed load of 50 KN/m under working condition. The effective span of beam is 5.5 m, M30 concrete and Fe 500 steel are used for the beam. |  |
| b) | An R.C. beam 230mm x 400mm (effective) is provided with reinforcements as mentioned below: Case Tensile steel Compression steel (i) 4 # 12mm 2 # 12mm (ii) 4 # 16mm 2 # 16mm Determine ultimate moment of resistance of beam in each case, if M25 concrete and Fe415 steel used in each case. |  |
| c) | A beam 230mm x 350mm deep consists of 4#16mm on tension side and 2#12mm on compression side. The effective cover of 35mm each is provided for compression and tensile steel. If M25 concrete and Fe415 steel are used, determine the safe udl the beam can support over an effective span of 5.5m under working condition. |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | Design a slab for a room having 4m x 5m as clear dimension. The slab is supported on 230mm wide beams alround. One long edge of slab is continuous and other three edges are discontinuous. M25 concrete and Fe415 steel is used. Take LL on slabs as 2KN/m2 and finishing load as 1KN/m2 | (8) |
|  | ### OR |  |
| a) | Design a longitudinal stair to carry a LL of 2.5KN/m2 and finishing load of 1KN/m2. The clear dimension of staircase is 2.4m x 4.8m. The floor to roof height is 3.2m. Use M25 concrete and Fe500 steel. | (8) |
| b) | Design a cantilever balcony slab 1.65m wide to carry a live load of 3KN/m2 and finishing load of 0.75KN/m2. Take M20 concrete and Fe415 steel. | (4) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a short circular column with lateral ties to carry an axial load of 500KN. The effective length of column is 3m. M25 concrete and Fe500 steel are used for column. |  |
| b) | Design a short rectangular column having effective length of 2.9m to carry an axial load of 600KN. M25 concrete and Fe415 steel are used for column. |  |
| c) | Design a square footing to carry a column load of 900KN. The size of column is 300mm x 300mm having 4#12mm as longitudinal reinforcement. The bearing capacity of soil is 175 KN/m2. Take M20 concrete and Fe415 steel for the footing and apply necessary checks for the footing designed. |  |

***********