## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Process-II(4236)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How are the shapers classified? |  |
| b) | List the different types of milling machines. |  |
| c) | How the size of the lathe is specified? |  |
| d) | What is cropping? List the machines used. |  |
| e) | Write note on portable nibbling machine. |  |
| f) | List the safety precautions to be followed while working on a guillotine machine. |  |
| g) | Write note on “feature of sharing”. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of sketch explain how the plate edge is prepared with the help of a machine. |  |
| b) | Explain the principle and procedure of plasma cutting. |  |
| c) | With the help of sketch explain how the circle is cut out of plate by gas cutting by hand. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write short note on - Gouging of weld - CNC cutting machine. |  |
| b) | List and draw sketches of different types of grinding wheels. |  |
| c) | Explain term A60H5V with reference to grinding wheel specification. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List any six differences between planer and shaper. |  |
| b) | Differentiate between “Down milling” & “Up milling” with the help of sketches. |  |
| c) | Write note on - Counter boring - Portable drilling machine. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a simple sketch of horizontal boring machine and name parts. |  |
| b) | List any four differences between “Three jaw self centering chuck” & “Four jaw independent chuck”. |  |
| c) | Explain the technique of cutting thick plates near the edges using flame cutting. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Job holding methods on shaping machine |  |
| b) | Punching holes |  |
| c) | Punch type nibbler |  |
| d) | Parallel shaft rotary shearing machine |  |
| e) | Straightening of Aluminium sheets |  |
| f) | Mechanical straightening |  |

***********