## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electrical Engg.

Subject: Electrical Engineering Materials(4101)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Why copper is used in the windings of electrical machines even though it is expensive? |  |
| b) | What are the 3 groups of resistor materials based on their practical applications? Mention the applications of each group. |  |
| c) | Mention any six practical applications of semiconductors. |  |
| d) | Define relative permittivity and absolute permittivity and give expression for each. |  |
| e) | Explain the terms: i) Volume resistance and ii) Surface resistance with respect to insulating materials. |  |
| f) | Define the following: i) Magnetic circuit ii) Permeability iii) Reluctance. |  |
| g) | What are thermocouples? State their applications. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List out the special properties of the following materials and give their area of applications: i) Mercury ii) Carbon. |  |
| b) | Give any two advantages of underground cables over overhead transmission lines. Give any six required properties for underground cables. |  |
| c) | Explain the meaning of the term super conductors and list some of the superconductor materials. Give their areas of applications. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the meaning of extrinsic and intrinsic semi conductor. Give examples of each. |  |
| b) | With a neat diagram explain selenium rectifiers and give their applications. |  |
| c) | State the merits of semiconductor devices used in electrical industry. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss in detail non polar and polar materials. |  |
| b) | - Define dielectric strength. State the factors affecting dielectric strength. - Define dielectric loss. State the factors affecting dielectric loss. |  |
| c) | Explain in brief conduction through liquid and solid dielectrics. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | List the properties and uses of the following gaseous insulators: i) Air ii) Hydrogen. |  |
| b) | Give the general classification of insulating materials giving examples for each. |  |
| c) | Briefly explain synthetic rubbers, their properties and applications. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the impurities in ferromagnetic materials and effects of these impurities on ferromagnetic materials. |  |
| b) | Briefly explain what do you mean by hysterisis loss and magnetostriction. |  |
| c) | List the properties and applications of phosphorescent materials and fluorescent materials. |  |

***********