## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mechanical Engg./E/FT

Subject: Refrigeration & Airconditioning(5034)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Steam tables and Psychrometric chart is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | With a neat sketch explain working of Vapour absorption system. |  |
| b) | Define one Tonne of Refrigeration. Give its value in KW. |  |
| c) | Represent a Vapour compression cycle on T-S diagram for following cases: i)Vapour is dry and saturated at end of compression ii)Vapour is wet after compression. |  |
| d) | Sketch the various parts of a domestic Refrigerator. |  |
| e) | State any six desirable properties of ideal refrigerant. |  |
| f) | State the advantages of water-cooled condensers over air-cooled condensers. |  |
| g) | Define the following terms (i)Specific Humidity (ii) D.B.T (iii) D.P.T. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Giving a neat sketch explain the parts of simple Vapour compression system. |  |
| b) | Draw a neat sketch of three fluid system of refrigeration (Electrolux refrigeration system) and explain its working. |  |
| c) | A refrigeration cycle works between 5C and 40C evaporator and condenser temperature respectively. Calculate C.O.P of cycle if refrigerant entering the compressor is 100% dry. Use the following table. Temp (C); Pressure (bar); Specific Volume (m3/kg); Enthalpy (KJ/Kg) Liquid Vapour; Entropy (KJ/KgK) Liquid Vapour. 5C; 3.69296; 0.04781; 204.72; 354.64; 1.0169; 1.556. 40C; 9.5944; 0.01836; 239.29; 368.67; 1.1324; 1.545. |  |
|  | Take Cp=0.69 KJ/KgK for Vapour and 0.96 KJ/KgK for liquid. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain construction and working of open type of compressor giving neat sketches. |  |
| b) | With neat sketch explain operation of Thermostatic expansion valve. |  |
| c) | With neat sketch explain a Natural Draft Cooling Tower. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The atmospheric air has 35C dry bulb temperature and 50% RH. Using Psychometric chart find: i) Wet Bulb temp ii) Dew point temp iii) Humidity ratio iv) Enthalpy of air per kg of dry air. |  |

………2/-

-2-

| b) | For a sample of air having 22C D.B.T; RH of 30% at barometric pressure of 760 mm of Hg, calculate (i) Vapour pressure ii) Specific Humidity iii) Vapour density iv) Enthalpy. (Use of steam tables is permitted) |  |
| --- | --- | --- |
| c) | What is Sensible Heating and Sensible Cooling? With help of simple sketch show process on Psychrometric chart. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With help of neat sketch explain working of Ice Plant. |  |
| b) | With help of neat sketch explain working of Window AC. |  |
| c) | Explain working of Air Handling Unit with help of neat sketch. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With help of neat sketch explain working of sling Psychrometer. |  |
| b) | What is the function of filter? How are filters classified? Explain any one in brief. |  |
| c) | Give requirements of good room air distribution system. |  |

***********