## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Automobile Engg./P

Subject: Machine Tools & Production Process-I(4050)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between Hand tools & Machine tools. |  |
| b) | How are Hand tools classified? |  |
| c) | What do you understand by ‘direct’ and ‘indirect’ measuring instruments? |  |
| d) | Name any three angle measuring tools and sketch any one of them. |  |
| e) | Explain with reference to a lathe: - Cutting speed - Feed. |  |
| f) | Calculate the facing time for a diameter of 60 mm, at a spindle speed of 80 rpm. Take cross feed = 0.2 mm/rev. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain drill point geometry giving neat sketch. |  |
| b) | Describe the construction and working of a micrometer screw gauge. |  |
| c) | Describe in brief any two marking tools. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly describe following parts of a lathe i) Head stock ii) Carriage. |  |
| b) | Describe any two work holding devices used on lathe. |  |
| c) | Describe following lathe operations: i) Knurling ii) Boring. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe procedure of Taper turning on a lathe. (any one method). |  |
| b) | List different cutting tools used on a lathe and describe any two of them. |  |
| c) | Name any two types of Tool post and describe any one in brief giving a sketch. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the describe properties of a cutting fluid. |  |
| b) | Explain the following with respect to a cutting tool material: i) Hot hardness ii) Wear resistance. |  |
| c) | Define machineability. List any three of the various criteria considered while evaluating machineability. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Taps & dies |  |
| b) | Vernier depth gauge |  |
| c) | Three jaw self-centering chuck |  |
| d) | Thread cutting on lathe |  |

***********