## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-I(4001)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the difference between plan & Geodetic survey. |  |
| b) | Give classification of survey based on object of survey |  |
| c) | Give conventional symbols for the following: (i) Cutting (ii) Marshy land (iii) railway. |  |
| d) | Define ranging. Explain method of direct ranging by line ranger. |  |
| e) | What is the difference between temporary and permanent adjustments of level? |  |
| f) | RL of BM is 100.00m of staff reading on BM is 1.285m. Calculate RL of line of sight. Also calculate RL of point B on which staff reading is 1.00m |  |
| g) | What is parallax? How can it be eliminated? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 30 m chain was tested before commencement of day’s work and was found to be correct after chaining 3000m, chain was found to be half decimeter too long. At the end of day’s work after chaining a total distance of 5400m, the chain was found to be one decimeter too long. What is the true distance chained? |  |
| b) | What is chain triangulation and what are the conditions to be fulfilled by survey stations/survey lines? |  |
| c) | Describe with sketches how an obstacle with interrupts ranging but not chaining is crossed over. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is closing error? How would your adjust it graphically? |  |
| b) | Draw a neat sketch of prismatic compass. List different parts and mention function of each part. |  |
| c) | Explain various methods for determining width of a river. |  |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Following bearing were observed with compass. Line; FB; BB. AB BC CD DE EA; 74 91 166 177 189; 254 271 343 0 9. At which station do you suspect local attraction? Find corrected bearing. |  |
| b) | - What is back bearing? What are advantages of observing back bearing in a traverse? - Convert following Q.B to WCB. (1) N 30 W (2) N 20 E (3) S30 W |  |

………2/-

-2-

| Q.No.5. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | Following consecutive readings were taken with level 1.904, 2.653, 3.906, 4.026, 1.964, 1.702, 1.592, 1.261, 2.542, 2.006, 3.145. Instrument was shifted after 4th & 7th reading, BM is 100m. Calculate RL of points by both methods and apply check. | (8) |
| b) | Explain the following types of leveling: - Simple leveling - Fly leveling. | (4) |
|  | ### OR |  |
| b) | Explain term: - Line of collimation (ii) Change point (iii) Reduced level (iv) Back sight. | (4) |
| Q.No.6. | Write short note on any three: | 3 x 4 = 12 |
| a) | Profile levelling |  |
| b) | Chain & compass traversing |  |
| c) | Optical square |  |
| d) | Box sextant |  |
| f) | Error in levelling |  |
| g) | Tilting level |  |

***********