## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electronics Engg./E/EI/COMP/EC/MED/E&EE

Subject: Digital Electronics-I(4134)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Find gray code of the following (56)10. |  |
| b) | Show positive and negative logic with the help of figures. |  |
| c) | Draw the symbol, truth table for T flip-flop and draw T flip-flop from JK flip-flop. |  |
| d) | State merits and demerits of CMOS logic family. |  |
| e) | Draw the diagram of a 4 bit SISO register using SR flip-flops. |  |
| f) | Convert the following to its Hexadecimal equivalent: (i) (1742.102)10 (ii) (242.32)10. |  |
| g) | Show that NAND logic does not follow associative law. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the diagrams to get AND, OR, NOR & NOT using only NAND gates. |  |
| b) | - Convert (156)10 to gray code and BCD code. - What is the specialty of gray code? |  |
| c) | Add the following using 2’s complement method. (i) (-54)+(-64) (ii) (+56) + (-66) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Simplify using K map. X = ACD+ACD+ABCD+ABCD+ABC |  |
| b) | Simplify the following and implement the given equation and the solution using gates. Y = (A + C)(A + D)(B + C)(B + D) |  |
| c) | Simplify and give the truth table for the following boolean expression. Y = A+B.BC+BC |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the diagram of JK flip-flop and explain its operation with truth table. Draw a diagram to modify JK flip-flop to a T flip-flop. |  |
| b) | Explain the working and give the truth table of RS Flip-flop. Explain how it can be used as a basic memory cell. |  |
| c) | What is race around condition in JK flip-flop? How it can be overcome? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat diagram of a SIPO register for storing 4 bits. Draw the timing diagram for 5 clock pulses and show transfer of data: (1101)2. |  |
| b) | Explain with neat diagram Johnson counter. Also draw timing diagram (4bit). |  |
| c) | Draw the diagram for 3-bit Asynchronous down counter and draw resulting timing diagram. |  |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Define the terms with respect to digital ICs - Speed of operation - Fan in - Fan out. |  |
| b) | Draw a circuit diagram of TTL NAND gate and write characteristics of TTL family. |  |
| c) | Draw and explain working of DTL NAND gate. |  |

***********