## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Resistance & Propulsion(4339)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Apparent slip |  |
| b) | Vertical Axis Propeller |  |
| c) | Stern tube |  |
| d) | Froude’s Number |  |
| e) | Reduction Gear box |  |
| f) | Residuary Resistance |  |
| Q.No.2. | Describe the purpose and procedure for testing a ship model in an experimental towing tank. How do you scale the results for a ship? | (12) |
| Q.No.3. | The calculated power for naked hull of a ship of length 119m and displacement 9700 tonnes is 1750 KW for a speed of 12 knots. Find the total resistance, frictional and residuary resistance components of the Ship. Assume Froude’s skin friction resistance coefficient = 1.422 & S=2.58ΔL. | (12) |
| Q.No.4. | Describe the phenomenon of cavitation in propellers. Why is its harmful to propellers? What is super cavitative propeller? | (12) |
| Q.No.5. | What is a screw propeller? Draw a typical propeller drawing and explain the various views. | (12) |
| Q.No.6. | Describe various losses which occur in the power transmission system from engine to the thrust generated by the propellor. What is meant by relative rotative efficiency? | (12) |
| Q.No.7. | Describe any two of the following: - Kort nozzle propeller ii) Controllable pitch propeller iii) Voith Schnieder propeller. | (12) |
| Q.No.8. | Explain why stern tube is provided in a ship. Draw the sketch of a stern tube and explain how sea water entry to the ship through stern tube is avoided. | (12) |
| Q.No.9. | With the help of neat sketches, explain i) Working of a Hydraulic clutch ii) Reversing function of a gear box. | (12) |

***********