## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electronics Engg./EC

Subject: Data Communication(5138)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | Differentiate between Asynchronous and synchronous type of communications. (Any 3 points) | (3) |
| b) | With a neat schematic differentiate between step-index and graded index optical fibers. (any two differences) | (3) |
| c) | Define Quantum efficiency and emission response time of an LED. | (2) |
| d) | Mention 2 types of photo detectors. | (1) |
| e) | Mention the basic sections of a satellite communication system. | (1) |
| f) | What are the three requirements to be fulfilled by a geosynchronous satellite? | (2) |
| g) | What is simplex, Half duplex and full duplex communication? | (3) |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Bus & Ring topologies. Also mention their advantages & disadvantages. |  |
| b) | List and explain the functions of network layer & physical layer. |  |
| c) | Explain the format of I-frame in a HDLC protocol. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is data transparency? How is it achieved in BSC protocol? |  |
| b) | Explain packet switching with a neat diagram. Briefly describe the two methods.x |  |
| c) | Describe classification of modems briefly. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the mechanism of signal transmission through an optical fiber. Also draw the structure of a single fiber and show its components. |  |
| b) | Explain scattering and bending losses in an optical fiber. |  |
| c) | Explain two types of fiber splicing methods with a neat diagram. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain outside vapour phase oxidation method of fabricating a fiber preform. |  |
| b) | Explain the construction of a PIN photodiode and explain its working. |  |
| c) | Answer any two: - List the three Kepler’s laws that govern the satellite motion. - Write three applications of satellite communication. - Write 2 advantages and 1 disadvantage of satellite communication. | 2 x 3 = 6 |

………2/-

-2-

| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain TDMA frame structure with a diagram. |  |
| b) | Explain a satellite transponder with a neat diagram. |  |
| c) | Describe the functioning of Altitude and orbit control subsystem. |  |

***********