## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Mechanical Engg./A/SB

Subject: Strength of Materials(4032)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following: (i) Hardness (ii) Toughness (iii) Ductility. |  |
| b) | Derive the strain energy stored in a body due to a gradually applied load. |  |
| c) | State and explain the Perpendicular Axis theorem. |  |
| d) | A copper wire of 5mm diameter is required to be wound around a drum. Find the minimum diameter of the drum if the stress in the wire is not to exceed 100MPa. Ecopper = 100 Gpa. |  |
| e) | A circular cast Iron column of diameter 200mm carries a vertical load of 500KN at a distance of 25mm from the axis. Find the maximum stress induced in the bar. |  |
| f) | A cylindrical shell of 2m diameter is made up of 10mm thick plates. Find the circumferential stress if the shell is subjected to an internal pressure of 3.5MPa. Efficiency = 75%. |  |
| g) | A steel rod 6m long and 40mm diameter is used as a column with one end fixed and the other end free. Determine the crippling load by Eulers formula. E = 200 GPa. |  |
| h) | A simply supported wooden beam 150mm wide and 200mm deep has a span of 5m. Determine the load that can be placed at its centre to cause the beam a deflection of 8mm. E = 6 GPa. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In a composite made of brass and steel, the steel tube is having 200mm ID and 10mm thickness. The outer brass tube is having 220mm ID & 12mm thickness. The two tubes are of the same length. The brass tube carries an axial load of 1000 KN. Find the stress and the load carried by each tube and also the amount it shortens. |  |
| b) | A metal bar of 40mm x 60mm section is subjected to an axial compressive load of 600KN. If the contraction of 200mm gauge length is found to be 0.6mm and increase in thickness as 0.05mm, find ‘E’ and ‘’ for the bar. |  |
| c) | A rod of steel 15m long is at a temperature of 18C. Find the free expansion of the rod when the temperature is raised to 58C. Also find the temperature stress produced when: - Expansion of the rod is prevented. - When the rod is permitted to expand by 6mm.  = 12 x 10–6/C and E = 2 x 105 N/mm2 |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A point in a strained material is subjected to a compressive stress of 80N/mm2 and a shear stress of 60N/mm2. Determine the maximum and minimum intensities of the direct stress. | ………2/- |

-2-

| b) | A weight of 200N is dropped on a collar attached to the end of a vertical bar 2 m long and 30mm in cross-section from a height of 250mm. Calculate the maximum instantaneous elongation and the stress produced in the section of the bar. E = 2 x 105 N/mm2 |  |
| --- | --- | --- |
| c) | A cantilever beam AB of span 2m is fixed at A and free at B. It carries point loads of 500N, 800N and 600N each at every 0.5 m form the fixed end. Calculate and draw the shear force and bending moment diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A beam 5m long is simply supported at A & B. It carries a 20KN load at a point 2m from the left end support and a uniformly distributed load of 10KN/m over the remainder of the beam. Calculate and draw the shear force and Bending moment diagram. |  |
| b) | Find the moment of Inertia of a ‘I’ section about the centroidal X and Y axis having the following dimensions: Top flange = 120mm x 15mm, Bottom flange = 200mm x 15mm & Web = 120mm x 20mm. |  |
| c) | A hollow square section with OD and ID of 60mm & 40mm respectively is used as a cantilever of span 1.5m. How much concentrated load can be applied at the free end if the maximum bending stress is not to exceed 40MPa. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A hollow column with OD 200mm and ID 175mm carries an eccentric load of 120N. Find the greatest eccentricity when the load can have without producing tension on the cross-section of the column. |  |
| b) | A solid shaft 120mm diameter transmits 130KW at 150 rpm. Find the maximum stress induced in the shaft. Also find the angle of twist in a length of 8m. G = 8 x 104N/mm2. |  |
| c) | A hollow shaft of diameter ratio 4/6 is to transmit 400KW at 100rpm with the maximum torque 30% greater than the mean torque. The shear stress is not to exceed 70 N/mm2 and the twist in a length of 5m is not to exceed 3. Calculate its OD and ID. G = 8.5 x 104N/mm2 |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A thin cylindrical shell of thickness 6mm and inner diameter 400mm is subjected to an internal pressure which produces a strain of 1/2700 in the diameter. Find this internal pressure and consequent hoop and longitudinal stress. E=2 x 105N/mm2 & =0.3. |  |
| b) | A hollow cast iron column whose OD = 250mm has a thickness of 20mm. It is 2m long and is fixed at both ends. Calculate the safe load by Rankines formula using a factor of safety = 4. Also calculate the slenderness ratio.  = 1/1600 and compressive stress = 500N/mm2 |  |
| c) | A beam has a rectangular section of 5m long and is simply supported at both ends. The beam is required to carry a load of 50 KN uniformly distributed over the entire span. Find the depth and width of the beam. Maximum bending stress is not to exceed 7 MPa and maximum deflection is limited to 10mm. E = 11 GPa. |  |

***********