## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Civil Engg./CT

Subject: Soil Mechanics(4012)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is soil? Explain three phase system. |  |
| b) | Define the following: i) Void ratio ii) Density Index - Porosity. |  |
| c) | Define compaction of soil. Enlist the factors controlling compaction. |  |
| d) | Define the following: i) Flow lines ii) Equipotential lines. |  |
| e) | Enlist the objectives of soil exploration. |  |
| f) | Explain the importance of geology as applied to various Civil Engineering projects/problems. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A soil sample has moisture content 28% and specific gravity is 2.6, the unit weight of soil is 1.65 gm/cc. Find void ratio, porosity and degree of saturation. |  |
| b) | Explain the grain size distribution curve. |  |
| c) | Explain the laboratory procedure for determining liquid limit. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define permeability and explain the factors controlling permeability of soil. |  |
| b) | The coefficient of permeability is calculated in the laboratory by falling head method. The data used and test results obtained are as follows. - Height of sample……………… … 200mm - Diameter of the sample …………. 70mm - Diameter of stand pipe ………….. 15mm - Initial head of water ……………… 600mm - Final head of water after 150 seconds …………………….. 300mm Find the coefficient of permeability in m/sec. |  |
| c) | Explain any two methods of compaction of soil in field. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how to find the maximum dry density dmax and optimum moisture content of given soil sample in the laboratory. |  |

………2/-

-2-

| b) | In a proctor compaction test the following readings are recorded. Capacity of proctor mould = 1000cc Water contents %; 13%; 15%; 18%; 20%; 24%. Weight of compacted soil (gms); 1500; 1700; 1780; 1650; 1600. Find the optimum moisture content and dry density. |  |
| --- | --- | --- |
| c) | Explain geophysical method of soil exploration. (any one method) |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the triaxial shear test on soil. |  |
| b) | The following results were obtained from a box shear test on a sandy clay specimen. Find the shear strength parameters. Cross-section of box (6cm x 6cm) Test No.; 1; 2; 3. Normal load (N); 280; 560; 1080. Shear force at failure (N); 240; 320; 460. |  |
| c) | What are unconformities? With a neat sketch explain any two types. |  |
| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| a) | Assumptions made in Rankines theory of earth pressure |  |
| b) | Effect of water table on bearing capacity of soil |  |
| c) | Disturbed and undisturbed soil sample |  |
| d) | Passive and active earth pressure |  |
| e) | Vane shear test |  |

***********