## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Food Technology

Subject: Food Engineering-I(4265)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any ten of the following Questions: | 10x1.5=15 |
| --- | --- | --- |
| a) | What is thermal conductivity? Give one example each of a good conductor and poor conductor of heat. |  |
| b) | Why are the tubes carrying the heating medium provided with fins? |  |
| c) | Explain the term “Equilibrium Moisture content”. |  |
| d) | State atleast two factors which affect falling rate period during dehydration. |  |
| e) | What is the limitation of Evaporative air coolers and why? |  |
| f) | Name two products that can be concentrated using falling film evaporators. |  |
| g) | Name two products dried using Drum Drier. Also state the limitations of drum drier. |  |
| h) | State the products frozen using Plate freezer. |  |
| i) | Explain the principle of steam distillation. |  |
| j) | State three important parameters required to be followed during frozen storage. |  |
| k) | List the components of a simple vapour compression system. |  |
| l) | Why is multi pass design required for heat exchangers? |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain thermal resistance to heat transfer by conduction. Explain the terms (i) Heat transfer coefficient (ii) Overall heat transfer coefficient. |  |
| b) | Name the heat exchanger you would choose for the following and why: - Pasteurization of milk - Heating of fruit juice - Heating of Tomato puree - Heating air in a drier. |  |
| c) | Explain the heat transfer by radiation. Explain the laws that govern them. Give two examples where heat transfer is by radiation. |  |
| d) | Explain with the help of graph, how to calculate Log mean temperature difference for counter and parallel flow in a heat exchanger. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4= 12 |
| a) | Explain the following terms: - Wet bulb temperature - Dew – point - Relative humidity - Saturated air. |  |
| b) | Suggest the driers used for the following and state the reasons. i) Instant Coffee ii) Diced Carrots iii) Peas iv) Orange juice. |  |
| c) | Explain the working and advantage of long tube evaporators. List the products concentrated using these evaporators. |  |
| d) | Compare the advantages and disadvantages of counter and parallel air flow in a drier. | …….2/- |
|  | -2- |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What are the advantages of Vacuum concentration of foods? Explain the working of steam jet ejector with a neat diagram. |  |
| b) | Tomato juice at 5 Brix is concentrated to 25 Brix. If 1000 kg of Tomato juice is used, how much of Tomato concentrate would you obtain? What equipment would you choose for this process? |  |
| c) | What are the advantages and limitations of the following driers: i) Spray drier ii) Drum drier iii) Cabinet drier iv) freeze drier. |  |
| d) | Explain the factors that affect the drying rate. What is case hardening? |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | With a neat diagram explain a simple vapour compression refrigeration system. |  |
| b) | Explain the working of a Blast freezer. Also explain how freezing rate is important to maintain quality of frozen foods. |  |
| c) | Explain how to calculate the refrigeration load for keeping fruits in a cold storage. |  |
| d) | Explain the working and uses of plate freezer. |  |
| Q.No.6. | Write short note on any three: | 3 x 4 = 12 |
| a) | Fractional distillation and its application |  |
| b) | Common refrigerants used –its merits and demerits |  |
| c) | Humidity measurement |  |
| d) | Spray drier and its parts |  |

***********