## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electrical Engg./E&EE

Subject: Substation Practice(5105)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is a Substation? List any four features an underground Substation should posses. |  |
| b) | Draw breaker and half bus bar arrangement and state its applications. |  |
| c) | List general specifications of C.T. |  |
| d) | What is an Isolator? Why it is used in Substations? |  |
| e) | With a neat diagram briefly explain importance of mimic diagram. |  |
| f) | List any three components of traction Substation & state their function. |  |
| g) | State importance of ‘alarm’ in a Substation. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Distinguish between indoor and outdoor Substation. Which type is normally preferred? Why? |  |
| b) | With neat diagram explain main & transfer bus bar scheme. State its advantages. |  |
| c) | Explain working principle of CT. How it is different from conventional transformer? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain single line diagram of a pole-mounted Substation. |  |
| b) | Why industrial loads have low power factor? Briefly explain any method of improving power factor. |  |
| c) | Calculate annual bill of a consumer whose maximum demand is 100KW, power factor is 0.8 lagging and load factor=70%. The tariff used is Rs.150 per KVA of maximum demand plus Rs. 3/- per units consumed. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List most commonly used devices for protection against lightning surges. Explain Rod gap arrester. |  |
| b) | Explain an isolated neutral system. List and explain its disadvantages. |  |
| c) | With a neat diagram explain Expulsion type lightning arrester. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With a neat diagram explain general construction of cable. |  |
| b) | List the methods of laying cables. Explain Draw-in system and state its advantages & disadvantages. |  |
| c) | Determine the size of cable required to carry maximum load current of 60A over a distance of 150m. The permissible voltage drop is 5% of 230V. Following cables are available. Size in mm2; Current Ratios A; Volt drop per ampere-meter. 25; 70; 2.5 mV. 35; 86; 1.8 mV. 50; 105; 1.3 mV. 75; 130; 0.93 mV. 120; 190; 0.54 mV. 150; 210; 0.45 mV. |  |
| Q.No.6. | Write a short note on any two: | 2 x 6 = 12 |
| a) | Care and maintenance of batteries in Substation |  |
| b) | Use of Silicagel & its reactivation |  |
| c) | Preventive maintenance of Circuit Breakers and Isolators |  |

***********