## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electrical Engg./E&EE

Subject: Switchgear & Protection(4110)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | i)State the types of current limiting reactors. ii)Define a fault. |  |
| b) | State the precautions to be taken during rewiring/ replacement of a rewirable fuse and H.R.C fuse. |  |
| c) | State the advantages & disadvantages of a static relay. |  |
| d) | Define 1. Protective zone 2. Main protection 3. Back up protection w.r.t. protective relay. |  |
| e) | Explain in brief the operation of an impedance relay. |  |
| f) | State the points to be considered during selection of a fuse for operation of static load and fluctuating load. |  |
| g) | Classify the faults & state the frequency of their occurrence. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief basic principle of operation of ELCB, with the help of neat circuit diagram. |  |
| b) | Explain the methods used for arc interruption in D.C. circuit breaker and explain each in brief. |  |
| c) | Explain in brief the constructional features of MOCB. |  |
| d) | Explain in brief principle of arc extinction in ACB. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State at least 8 merits & 4 merits of vacuum circuit breaker. |  |
| b) | Explain in brief principle of arc extinction in SF6 circuit breaker. |  |
| c) | What is the importance of resistance switching in air blast circuit breaker? How can it be achieved? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief the working of a bimetallic relay with the help of a neat diagram. State the type of fault against which it provides protection. |  |
| b) | Explain in brief constructional features of an IDMT relay. |  |
| c) | Draw a neat diagram of directional type overcurrent relay and label its parts. Give at least two points of distinction between a directional type overcurrent relay & non directional type overcurrent relay. |  |
| d) | An IDMT overcurrent relay has a plug setting of 150% and has a time setting multiplier of 0.5. The relay is connected in the circuit through a C.T. having ratio of 500:5 amps. Calculate the operating time of relay if the fault current is 6000 A. Following operating characteristic is given. PSM; 2; 3; 5; 8; 10; 15. Time for Tsm=1; 10; 6; 4.5; 3.2; 3; 2.5. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw a neat circuit diagram of using Merz Price circulating current scheme of protection in case of star-delta connected transformer and label its parts. Also state the difficulties of such scheme. |  |
| b) | State the type of protection provided for an alternator against unbalanced loading. Explain such protective scheme used, with the help of neat labelled circuit diagram in brief. |  |
| c) | Draw a neat circuit diagram to indicate use of percentage differential protection scheme for star connected stator winding of an alternator. Against which fault does it protect the alternator? |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i)List out the various types of faults one comes across in three phase Induction motor. |  |
|  | ii)State the effects of single phasing condition occurring in 3 phase Induction motor. Draw a neat circuit diagram to indicate a protective scheme used to protect it against single phasing. |  |
| b) | Explain in brief with the help of neat circuit diagram, starter earth fault protective scheme provided for 3 phase Induction motor. |  |
| c) | State the types of protective schemes used for protection of busbar against earth fault. Explain in brief any one of such scheme of your choice with the help of neat circuit diagram. |  |

***********