## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Architectural Engg.

Subject: Structures-I(4537)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How force differs from mass of an object? |  |
| b) | Explain the effects of force on a body. |  |
| c) | Explain the concept of transverse loads in producing bending moments and shear force. |  |
| d) | List down various types of steel and their specific use. |  |
| e) | Sketch and label various parts of (1) An arch and (2) A Vault. |  |
| f) | State various types of beams with different end conditions. |  |
| g) | Explain structural cracks in a building with examples. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the principle of Resolution of forces. The following forces act at a point - 20N inclined at 30 to the North of east - 25N towards North - 30N towards North of west. Find the magnitude and direction of resultant force. |  |
| b) | What is couple? With a neat sketch explain classification of couple and state characteristics of couple. |  |
| c) | State and explain the difference between the Nominal mix proportions and Design mix proportions as per Indian standards. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain ‘Concrete Mix Design’ with various steps involved in it as per Indian standard. |  |
| b) | Differentiate between: - Short column & Long column - Limit state method & working stress method of design. |  |
| c) | State guidelines for design of load bearing walls with material and design considerations. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cantilever beam carries an udl of 12KN/m throughout its span and a concentrated load of 35KN at its free end. If the span of beam is 7m, draw SFD & BMD for the beam. |  |
| b) | A simply supported beam of 8m span carries two concentrated loads of 20KN & 40 KN at distance of 3m & 5m respectively from left support. Draw SFD & BMD for the beam. |  |
| c) | Find the centroid of a T-section having flange 250mm x 20mm & web 200mm x 12mm. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With neat sketches explain critical sections of Bending moment and shear force for (i) simply supported beam (ii) Continuous beam (iii) Fixed beam. |  |
| b) | State the causes of cracks in a building and explain any four causes in details. |  |
| c) | Explain how the cracks in building can be prevented by adopting proper architectural design and foundation design. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Post beam type and space truss type of system in a structure |  |
| b) | Centre of gravity and Centroid |  |
| c) | Stresses in structure due to torsion and bending |  |
| d) | Tensile and shear cracks in a building |  |
| e) | Non structural cracks in a building |  |

***********