## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electronics Engg./EC/EI/COMP/MED/E&EE

Subject: Microprocessors(8085)-I[4142]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the organisation of microprocessor based system with a neat diagram. |  |
| b) | Explain any 3 peripheral or externally initiated operations of microprocessor. |  |
| c) | List the classification of instruction set based on word size and give examples for each. |  |
| d) | Explain the following instructions with example. i) ORA C ii) NOP |  |
| e) | Draw the interpretation of the accumulator bit pattern for the sim instruction. |  |
| f) | Specify the memory location and its contents after the following instructions are executed. LXI B, F7FFH MOV A,B INX B STA 7075H HLT |  |
| g) | What is the content of stack pointer, register pair HL & DE after the execution of the following. LXI SP, 4020H LXI H, 2050H LXI D , 5555H PUSH H POP D |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the programming register model of P-8085. |  |
| b) | Interface a 4K x 8 EPROM using 3x8 decoder to P -8085. Draw the interfacing circuit and specify its memory address range. |  |
| c) | With one example each, explain any six branching instructions available for programming P -8085. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With examples, explain the 16 bit instruction to - Load 16 bit data in register pair - Load 8 bit data directly in memory location - Transfer (copy) data from P to memory location. |  |
| b) | Define the followings: i) Instruction cycle ii) Machine cycle iii) T State iv)Mnemonic v) Machine language vi) Assembly language |  |
| c) | Using example of loop within loop technique for time delay, describe the delay calculations for the same. Also draw flowchart denoting the technique. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Write a program to count the number of even data bytes in a block of 15 data, starting at memory location 7500H. |  |
| b) | Specify the number of times the following loops are executed. i) UP: MVI B,64H ii) ORA A iii) MVI A,17H NOP MVI B,64H LP: ORA A DCR B LP: DCR B RRC JNZ UP JNC LP JNC LP |  |
| c) | Write a program to find the size of a block with starting address 7550H. The end of the block is indicated by the byte FFH. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the call instruction in detail, illustrating the function of each of its machine cycles. |  |
| b) | Explain the function of the following instructions i) PUSH PSW ii) SPHL iii) POP H |  |
| c) | Using schematic diagram, explain the implementation of non- vectored interrupt. (Assume that RST 6 is loaded on the data bus when interrupt occurs.) |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain two advanced subroutine concepts used in P-8085. |  |
| b) | Explain vectored interrupt in brief. |  |
| c) | Briefly describe the steps involved in P-8085 interrupt process. |  |
| d) | Explain the communication lines used by P -8085 |  |

***********