## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Drawing & Calculations(4346)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Dynamic stability |  |
| b) | Centre of flotation and longitudinal centre of buoyancy |  |
| c) | Margine line and margine plate |  |
| d) | Classification society |  |
| Q.No.2. | The displacement of M.V. tanker is 24700 tonnes and KG=10m. Construct a curve of statical stability. From the curve find i) Maximum GZ ii) Angle of heel at which maximum GZ occurs iii) Range of stability iv) Approximate GM initial v) Moment of statical stability at 40 heel. Refer data sheet in Fig.1. | (12) |
| Q.No.3. | Answer the following: |  |
| a) | A box shaped vessel 64m x 10m x 6m floats in salt water on an even keel at 5m draught. A forward compartment 6m long and 10m wide extends from the outer bottom to a height of 3.5m, and is full of cargo of permeability 25%. Find new draught if the compartment is now bilged. | (08) |
| b) | Define permeability, broken stowage and stowage factor. | (04) |
| Q.No.4. | Answer the following: |  |
| a) | Sketch and describe stiffened bulkhead. | (08) |
| b) | Draw a neat sketch to show how watertightness is maintained in pipe penetration and longitudinal structural member penetration of transverse bulkhead. | (04) |
| Q.No.5. | Explain with neat sketch procedure to obtain GZ cross curves of stability for different displacements of the ship. | (12) |
| Q.No.6. | Answer the following: |  |
| a) | With neat sketch explain the procedure to obtain floodable length curve. | (08) |
| b) | With neat sketch derive the equation to find GZ from KM cross curves of stability. | (04) |
| Q.No.7. | With neat sketch explain how statical stability curves are obtained from GZ cross curves of stability. | (12) |