## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Electrical Engg.

Subject: Illumination Engineering(5106)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define (1) Natural lighting (2) Artificial lighting. |  |
| b) | State the type lighting system used in automobile head lights and justify yours choice. |  |
| c) | Give three points of distinction between Luminance and illuminance. |  |
| d) | State the law of illumination applicable for determination of illumination level at a point on working plane which is exactly below the lighting source hung from ceiling of a room interior. Also state the expression for illumination level and indicate clearly each term used in expression. |  |
| e) | Define the following: (1)Maintenance factor (2)Uniformity ratio. Also state the range of their nominal values. |  |
| f) | In case of street lighting, define (1) Kerb (2) Dual Carriage way (3) Outreach, |  |
| g) | State the requirements of a good street lighting. |  |
| h) | Define the following: i)Waste light factor ii) Beam factor. For which lighting are such factors applicable. Also state nominal value of Waste light factor for statue. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - Hard shadows are to be minimized but soft shadows are to be preserved. Say true or false and justify it. - Define colour rendition. Arrange the following lamps in ascending order of their colour rendition. - Sodium vapour lamp (ii) Incandescent lamp (iii) Fluorescent tube (iv) CFL. - State the types of glares. Can they be minimized. state the factors affecting glare. 4) State the types of lamps exhibiting stroboscopic effect. |  |
| b) | State the different types of lighting schemes. Also state two applications of each of these schemes. Explain each scheme in brief. |  |
| c) | State the different types of reflections we come across in Illumination Engineering. Define each of them and give one example of luminaire exhibiting each of these reflections. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the following for interior lighting design: - DLOR (ii) ULOR (iii) COU. State the different factors affecting selection of C.O.U. Also state the factors affecting selection of luminaries used in interior lighting. |  |
| b) | State all the requirements for indoor lighting design. Also give all the steps involved in the design |  |
| c) | State all the special features of entrance lighting, staircase lighting and corridor lighting. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the following: (1) Luminous flux (2) Luminous intensity (3) Horizontal polar curve (4) Vertical polar curve. |  |
| b) | A Lamp having candle of 200 in all directions is provided with reflector which directs 60 % of the total light uniformly on a circular area 20 m in diameter. The lamp is hung 12 m above this area. Calculate the illumination level at the edge of working plane and at its centre with reflector. |  |
| c) | For a Fluorescent tube mounted at a height of ‘H meters’ from the working plane and having luminous intensity of Io candles per meter length of the tube and for an aspect angle  specified in radians, state the expression for 1)the illumination level at a point on working plane which is exactly below one of the edges of a tube. 2) Illumination level at a point on working plane which is exactly below the centre of tube. 3) Illumination level at a point on the working plane which is located far away from one end of the tube. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the different types of street light luminaires. Also state the basis of their classification. Also state one application of each type of luminaires. Arrange these luminaires in ascending order of their presence of glare. |  |
| b) | List the steps involved in the design of street lighting. Also state the factors affecting the choice of luminaires used in street lighting. Also give the nominal value of maximum space to mounting height ratio for each luminaire. |  |
| c) | Give four features of sports lighting. |  |
| d) | Give four features of spot lighting. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State all steps required in flood lighting design. |  |
| b) | State the different location of flood light projectors. Draw neat sketches indicating their locations. |  |
| c) | Design an economical flood lighting scheme for a storage yard of 30m x 15m to be provided an illumination level of 150 Lux, using steel tubular poles of 12m mounting height. The flood light fittings of following photometric data are available. Round fitting with 250 W HPSV Lamp with beam angle of 2 x 25 and beam factor of 0.5. Square fitting with 2 x 250W HPSV lamp with beam angle of 2x14 and 2x40 with beam factor of 0.6. |  |

***********