## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-I(4054)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Why compression ratio of diesel engine is higher? |  |
| b) | List three qualities of diesel. |  |
| c) | What is bio-diesel? |  |
| d) | How cooling is done in a wet liner? |  |
| e) | What is piston slap? |  |
| f) | What is a thermostat valve? |  |
| g) | What is SAE number? |  |
| h) | Describe ignition lag. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How does a 2-stroke engine work? |  |
| b) | Explain the use of a timing chain. |  |
| c) | Explain with diagram various types of piston rings. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw crankshaft and explain its various parts. |  |
| b) | What is a sodium-cooled valve? |  |
| c) | A single cylinder 4 stroke engine has cylinder diameter 20 cm and stroke 30 cm. It operates at 3000 rpm. Mean affective pressure is 200KPa. Calculate the indicated power of the engine. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a theoretical P-V diagram for a four stroke diesel engine. Why is the actual diagram different from the theoretical? |  |
| b) | What is diesel knock? |  |
| c) | How is refining of crude petroleum done? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain heat balance sheet in a petrol engine. |  |
| b) | How does a radiator pressure cap work? |  |
| c) | What is the expansion reservoir that is used for the radiator? |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is diesel fuel injection pump calibration? |  |
| b) | Using neat diagram show MPFI system for a petrol engine. |  |
| c) | What is pressurized lubricating system? |  |

***********