## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2013 Examinations

Programme: Shipbuilding Engg.

Subject: Introduction to Thermodynamics(4334)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State and explain the second law of thermodynamics. |  |
| b) | What are the basic gas processes? How are they shown graphically on P-V diagram? |  |
| c) | List the different system functions of IC engines and state the primary function of each of them. |  |
| d) | Determine the dryness fraction of steam if 0.85 Kg of water is in suspension with 44 Kg of dry steam. |  |
| e) | What is a counter flow heat exchanger? |  |
| f) | Summarize the applications of refrigeration. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - The pressure of gas supplied to an engine is measured as 100mm of water gauge. If barometer reads 730mm of mercury. Determine the absolute pressure of gas in KPa. - 0.5 Kg of a gas is heated at constant volume from 25C to 375C. How many KJ of heat is supplied? Take Kv = 0.728 KJ/KgK. |  |
| b) | Establish the relation between gas constant R and two specific heats, Kp and Kv of a gas. |  |
| c) | State the difference between an adiabatic process and an isothermal process. 0.12m3 of air at 1.5 MPa and temperature expands adiabatically to pressure of 175 KPa. Find the final temperature and the work done. Take Kp = 1.0035 KJ/Kgk and Kv = 0.7165 KJ/KgK |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A quantity of gas at an initial condition of 14bar, 0.08m3 and 450K expands at constant pressure to 0.25 m3. Find i) final temperature ii) change in internal energy iii) mass of the gas iv) change in enthalpy. Take R = 0.287 KJ/KgK, Kv= 0.711 KJ/KgK |  |
| b) | Represent the air standard diesel cycle on a PV and TS diagram. Derive an expression for the diesel cycle efficiency. |  |
| c) | An engine working on the Otto cycle has cylinder diameter 100mm and stroke 150mm. The clearance volume is 0.295 x 10-3 m3, find the air standard efficiency. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how the wet steam, dry saturated steam and superheated steam is produced with the help of temperature enthalpy diagram. |  |
| b) | A boiler is supplying steam at a pressure of 9 bar and 90% dry. Find (i) the mass of 5m3 of this steam (ii) the enthalpy per m3 of this steam above 0C. |  |

………2/-

-2-

| c) | - Find the volume of one kg of steam at a pressure of 15 bar and 90% dry. - Determine the condition of steam at a pressure of 10 bar and temperature 200C produced from water at 0C. - A closed vessel contains 1.5m3 of dry saturated steam at 15 bar. What is the temperature and mass of this steam? |  |
| --- | --- | --- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain the Fourier’s law of heat. Give the applications of heat exchangers. |  |
| b) | Explain the following terms with reference to vapour compression refrigeration cycle: - Tonne of refrigeration - Coefficient of performance - Functions of the basic components in the refrigeration cycle. |  |
| c) | A R-12 refrigeration machine has a section temperature of 5C and discharge temperature of 40C. If the vapour entering the compressor is 98% dry and leaves the compressor 100% dry, Represent the cycle on a p-h chart and determine the COP of the cycle. Use R-12 refrigeration chart. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Compare two stroke cycle engines and four stroke cycle engines. |  |
| b) | Explain the four stroke cycle petrol engine. |  |
| c) | A four cylinder four stroke cycle petrol engine is to be designed to give a brake power of 185 KW at 35 rps. The stroke bore ratio is to be 1.5 to 1. Assuming a mechanical efficiency of 75% and indicated mean effective pressure of 830 KPa, determine the required bore and stroke. |  |
| d) | A furnace wall is made of refractory bricks of 350mm thick. The temperature difference across the wall is 800C. Find the heat loss per square meter per hour. Take K=4.5 W/mK. |  |

***********